Chapter 9: DBMH Analysis: Appendices

# Table of contents

- Appendix 9.A: The Satterthwaite degree of freedom

- Appendix 9.B: Demonstration of significance testing formulae

- Appendix 9.C: Text output file listing

- Appendix 9.D: Excel ANOVA tables

- Appendix 9.E: Code for validating DBMH analysis

- Appendix 9.F: Simulators for validating fixed-reader and fixed-case analyses

- Appendix 9.G: Code for meaning of pseudovalues

- Appendix 9.H: Testing for interactions

- References

# Appendix 9.A: The Satterthwaite degree of freedom

This Appendix is for those who have asked the author questions like "why is ddf not an integer" and "what is the Satterthwaite approximation"? Since in the author's opinion this is not adequately dealt with in the ROC literature, the following explanation is based on the original papers by Satterthwaite1,2, dating to the early 1940s.

Suppose  is a continuous random variable sampled from a zero-mean normal distribution with variance :

.

Letbe the total number of independent realized samples , . The (unbiased) estimate of sample variance  is defined by:

.

Satterthwaite calls a simple estimate of variance; it has degrees of freedom N - 1.  Let  denoted the corresponding random variable (after all, an estimate is a realization of a random variable). It can be shown3 that  follows a chi-square distribution with  degrees of freedom (Theorem 7.3.3 in “An Introduction to Mathematical Statistics and Its Applications” and see software demonstration below, file mainLarsenMarx455.R):

.

Therefore the distribution of is:

.

Since the mean of a chi-square distribution with  degrees of freedom is  (7.3.7 ibid) it follows that the expected value of  is , i.e., . Therefore,

.

Consider the distribution of a linear combination of simple estimates of variance, i.e., the problem addressed by Satterthwaite. Let and  denote two simple estimates of variance with expected values and , respectively, and corresponding (integer) degrees of freedom  and . This implies that  is based on observations, and likewise  is based on observations. Consider the sum:

.

It turns out that even for this simplest of cases the distribution of , while computable, is too complex for practical use (while true in 1941 it is probably not true now). Satterthwaite's idea was to approximate the exact distribution with a chi-square distribution with an appropriately chosen degree of freedom , not necessarily restricted to integers (subscript Sat = Satterthwaite). To determine  he imposed the condition that both the theoretical and approximate distributions of have the same variance. Since  is itself an estimate of variance, one is entering Alice's world of a possibly endless quest of estimating the variance of a variance. The variance of  is given by (see explanation below):

.

This equation is derived as follows (Satterthwaite's 1941 paper is missing a factor of two which does not affect the final result). From Eqn.  the distribution of  (i  = 1,2) is:

.

One does not need to subtract 1 as the degree of freedom is based on observations. Eqn.  implies:

.

Since the variance of a chi-square distribution is twice the number of degrees of freedom (7.3.7 ibid),

.

Finally, and here comes the great leap of faith, and analogous to Eqn. , the hope is that for :

.

Since the sum of variances of independent random variables is the sum of the individual variances:

.

This implies (the twos cancel):

.

Since the expected values are generally unknown (one would need the distributions of the variables, and the ability to perform the necessary integrations to calculate the expected values) in practice one uses estimates (i.e., observed values) in place of expected values:

.

[The physicist in the author thinks of this as adding resistors in parallel.] This equation allows one to estimate (do not be surprised if is not an integer – an integer value would be an incredible coincidence), specifically:

.

Eqn.  is readily extended to a linear combination of several simple estimates of variance:

.

Using estimates (i.e., observed values) in place of expected values:

.

The mean squares calculated in ANOVA are variance-like quantities, so the Satterthwaite approximation can be applied to them:

.

This approximation is relevant in the DBM F-statistic for testing the null hypothesis that the treatment effect is zero (ignoring the Hillis modification for now):

.

The distribution of this statistic under the null hypothesis is:

.

The numerator degree of freedom is  - no approximation needed here. The denominator is a linear combination of mean squares, with degrees of freedom (in order of the appearance of the terms in the denominator): ,  and , respectively. Therefore, applying Satterthwaite's approximation, on obtains:

.

### Appendix 9.A.1 Code illustration of Eqn.

First the author illustrates the theorem implicit in Eqn. , namely,  follows the chi-square distribution with N-1 degrees of freedom, where N is the number of samples over which the variance is estimated. The filename is mainLarsenMarx455.R, reflecting the page number of the book, where this theorem is to be found3 .

### Appendix 9.A.2 Code Explanation

Line 9 sets the true value of to 0.4. Line 10 sets , the number of samples over which the individual estimates of variances was obtained, to 10. Line 12 sets the number of simulations S to 1000, which is the number of independent trials for each of which the variance of 10 samples from  will be estimated. Line 14-15 creates arrays for holding the S values of Var and scaledS2, where scaledS2 represents the left hand side of Eqn. , i.e. the variable that is expected to be distributed  . Lines 18-19 perform the sampling, variance estimation and the assignments. For example, line 18 uses rnorm() to obtain N samples from a normal distribution with standard deviation sigma = 0.4, and then calculates the variance of the samples, which is assigned to Var[s]. The next line assigns the properly scaled variance estimate to scaledS2[s]. Line 23-26 shows the preferred way of visually testing a model, namely a quantile-quantile or Q-Q plot4. Line 23 assigns the 1000 element array scaledS2 to x. Line 24 defines the qqData frame.The next two lines calls the ggplot() function. Insert a break point at line 29 and clicking on Source yielding Fig. 9.A.1 (A). The overlaid dotted straight line, created by the geom_abline() option to ggplot(), has unit slope and passes through the origin. The visual quality of the fit is excellent. This shows that the scaled variance, namely the left hand side of Eqn.  indeed behaves like a chi-square random variable with N - 1 = 9 degrees of freedom.

| (A) | (B) |
| --- | --- |

Fig. 9.A.1: (A) Q-Q plot showing fit of left hand side of Eqn.  against a chi-square random variable with 9 degrees of freedom. (B) Histogram of left hand side of Eqn.  with superposed pdf of chi-squared distribution with df = 9. These plots were generated by

Click Continue to execute the rest of the code. These generate the corresponding histogram with the overlaid pdf, Fig. 9.A.1 (B). Again, the fit looks good, but perhaps not as compelling as the presentation in Fig. 9.A.1 (A). If a quantitative goodness of fit statistic is desired, the starting point would be the histogram, splitting up the x-axis into bins, and computing the chi-square goodness of fit statistic by comparing actual counts to expected counts.

### Appendix 9.A.3 Code illustration of Satterthwaite approximation

Open the file mainSatterthwaite1941.R.

### Appendix 9.A.3.1 Code Explanation

Line 5 defines  and , line 6 defines the corresponding number of samples  and : if not set to these values, do set them to the stated values. Line 7 prints these values, line 8 defines the degrees of freedom  and , line 10-11 calculate the "simple estimate" variances s1Sqd and s2Sqd, corresponding to and . Line 12 implements the Satterthwaite approximation Eqn. , where rSat is the Satterthwaite degrees of freedom. Line 13-14 prints rSat and the quantity:

.

According to Satterthwaite, this quantity, termed "approx. accuracy", should be close to unity (assuming it is larger than one; if not use its reciprocal) if the approximation is to have a reasonable chance of success. The approximation also gets better the larger the values of  and . Clicking on Source yields the following output:

### Appendix 9.A.3.2 Code Output

> source(...)

sigma1 =  1 sigma2 =  4 N1 =  5 N2 =  2

Satterthwaite df =  1.138241 , approx. accuracy (should be close to unity) =  59.26727

For the chosen values, the approximation is expected to be quite poor; approx. accuracy = 59.3, which is much larger than one. Line 16-26 sets up simulations to visually test the quality of the approximation. Line 19 obtains N1 samples from a zero-mean normal distribution with standard deviation sigma1, computes the variance of the sampled values, and assigns the result to v1[s], where s is the simulation index, ranging from 1 to S = 1000. Line 20 obtains N2 samples from a normal distribution with standard deviation sigma2, computes the variance of the sampled values, and assigns the result to v2[s]. Line 22 computes the sum of the two arrays v1 and v2, each containing simple estimates of variance, and scales it appropriately for comparison to a chi-square distribution, and assigns the result to scaledS2. [One is using the left hand side of Eqn. , i.e., multiplying the sum of v1 and v2 by rSat and dividing by the true variance of the sum] Line 22 implements the Q-Q plot. Sourcing this file yields Fig. 9.A.2 (A), evidence of a poor fit. Now change line 4 to read N1 <- 15;N2 <- 20 and source the file. Under these conditions the fit is quite good, Fig. 9.A.2 (B), even though approx. accuracy = 8.5 is not that close to unity.

| (A) | (B) |
| --- | --- |

Fig. 9.A.2: (A) Q-Q plot showing quality of Satterthwaite approximation with N1 = 5 and N2 = 2; the approximation is very poor. (B) Q-Q plot showing quality of Satterthwaite approximation with N1 = 15 and N2 = 20; the approximation is good.

# Appendix 9.B: Demonstration of significance testing formulae

The following code implements the significance testing formulae. It is the longer version of the code in mainDBMHBrief.R, which allows one to see what is going on and thus gain better understanding of the analysis. It still uses the RJafroc package to perform some of the tasks, e.g., reading an input file, calculating pseudovalues and mean squares, etc., but the rest of the analysis is shown more transparently. Open the file mainDBMH.R.

### Appendix 9.B.1: Code Explanation

Line 2 loads the RJafroc package. Line 4 defines the maximum allowed Type I error rate . Line 6 defines fileName as the Van Dyke dataset "VanDyke.lrc". Line 7 uses the RJafroc function DfReadDataFile() to read the contents of the Van Dyke dataset and line 8 calculates the pseudovalues, function UtilPseudoValues(), using the empirical AUC as the FOM. Line 12 calculates the mean squares, using Eqn. (9.12), implemented in the function UtilMeanSquares(). Line 13 calculates, using the function UtilFigureOfMerit (), the I x J array of FOM values.

Put a breakpoint (red dot) to the left of line 15 and click on Source. Try printing out some values (highlight variable followed by Run), using the following as a guide (some lines are not shown, and the output has been "vertically compressed" by deleting empty lines).

### Appendix 9.B.2: Code Snippets

Browse[2]> FOM

Rdr - 0   Rdr - 1   Rdr - 2   Rdr - 3   Rdr - 4

Trt - 0 0.9196457 0.8587762 0.9038647 0.9731079 0.8297907

Trt - 1 0.9478261 0.9053140 0.9217391 0.9993559 0.9299517

Browse[2]> retMS

$msT

[1] 0.5467634

$msR

[1] 0.4373268

$msC

[1] 0.3968699

$msTR

[1] 0.06281749

$msTC

[1] 0.09984808

$msRC

[1] 0.06450106

$msTRC

[1] 0.0399716

…

The variable retMS contains the desired mean squares.

Line 15 prints out a message signaling the beginning of random-reader random-case analysis. Line 16 calculates the denominator of the F-statistic, by implementing the denominator of Eqn. (9.23). The variable MS_DEN_DIFF_FOM_RRRC is an abbreviation for "mean square denominator difference-FOM random reader random case". Line 17 calculates the observed value of the F-statistic, Eqn. (9.23), denoted F_DBMH. Line 18 trivially calculates ndf and line 19 implements Eqn. (9.24) to calculate  and line 20 prints out . Line 21-22 calculates the critical value of the F-statistic and prints it and line 23 calculates the p-value and prints it.

Click on Stop to exit debug mode. Remove the breakpoint at line 15 (by clicking on the red dot; it should disappear); insert a new breakpoint at line 25 and click on Source to execute the entire file.

> debugSource(...)

Random reader random case analysis

Hillis ddfH =  15.25967

F statistic is  4.456319 , the critical value of F is  4.529639

p-value is  0.05166569

…

The observed F-statistic, 4.46, is slightly smaller than the critical value, 4.530, consistent with the observed p-value (0.0517) being slightly greater than 0.05. The same values were obtained earlier using the code in mainDBMHBrief.R. For this data set the p-value is not significant.

Line 25 -26 initializes the length I array trtMeans to the reader averaged FOMs for each treatment (implemented using the mean() function). Click repeatedly on Next until the cursor gets to line 27, then highlight trtMeans and click on Run:

### Appendix 9.B.3: Code Snippets

Browse[2]> trtMeans

[1] 0.8970370 0.9408374

These are the reader averaged empirical AUCs for the two treatments. Lines 28 – 33 calculates inter-treatment FOM differences; in the current example there is only one difference, that between treatment 1 and treatment 2. The longer code is needed to handle more than two treatments; the array dimensioning is wasteful of memory, but simpler to follow. Line 33 extracts differences where they exist (the remaining elements of the I x I trtDiff array are filled with NAs). Line 35 calculates the multiplier of the term in Eqn. (9.31), called std_DIFF_FOM_RRRC, an abbreviation for "standard deviation difference-FOM random-reader random-case". The variable CI_DIFF_FOM_RRRC (for "95% confidence interval difference FOM random-reader random-case") is an nDiffs by 3 array, where nDiffs is defined at line 36 (= I(I-1)/2 = one for I = 2) and for each difference there are three terms, the 1st is the value of the difference (the lower numeric indexed treatment minus the higher numeric indexed treatment), the 2nd is the lower limit and the 3rd is the upper limit of the 95% confidence interval. These are calculated in lines 39 – 41, where the last two lines perform the necessary multiplications by  shown in Eqn.  (9.31). Line 42-43 prints out the value of the difference and the lower and upper 95% CIs.  Exit Debug mode by clicking on Stop, remove the breakpoint at line 25 insert one at line 46, and click on Source. Some lines are suppressed in the ensuing code output.

### Appendix 9.B.4: Code Output

> debugSource(...)

Random reader random case analysis

Hillis ddfH =  15.25967

F statistic is  4.456319 , the critical value of F is  4.529639

p-value is  0.05166569

mean diff is  -0.04380032  and 95% CI is  -0.0879595 0.0003588544

Highlight CI_DIFF_FOM_RRRC and click Run.

### Appendix 9.B.5: Code Snippets

Browse[2]> CI_DIFF_FOM_RRRC

[,1]       [,2]         [,3]

[1,] -0.04380032 -0.0879595 0.0003588544

The 1st is the value of the FOM difference ("1-2"), the 2nd is the lower limit and the 3rd is the upper limit of the 95% confidence interval. Compare the two code snippets above for consistency.

Lines 46 – 54 perform fixed-reader random-case analysis. Line 48 calculates the F-statistic, where the numerator is the same as before, i.e., MST, but the denominator is MSTC, see Eqn. (9.38). In the code the denominator is called MS_DEN_DIFF_FOM_FRRC (abbreviation for "mean square denominator difference-FOM fixed reader random case"). Line 52 - 54 calculates the critical value of the F-distribution and the p-value, and prints them. Lines 56 – 65 calculates the reader-averaged treatment differences and the upper and lower limits of the CIs.

Lines 67– 86 perform random-reader fixed-case analysis. Click on Stop, remove any existing breakpoints, Ctrl-L to clear the Console screen, and click on Source:

### Appendix 9.B.6: Code Output

> source(...)

Random reader random case analysis

Hillis ddfH =  15.25967

F statistic is  4.456319 , the critical value of F is  4.529639

p-value is  0.05166569

mean diff is  -0.04380032  and 95% CI is  -0.0879595 0.0003588544

Fixed reader random case analysis

ddf =  113

F statistic is  5.475953 and critical value of F is  3.925076

p-value is  0.02103497

mean diff is  -0.04380032  and 95% CI is  -0.08088303 -0.006717613

Random reader fixed case analysis

ddf =  4

F statistic is  8.704 and critical value of F is  7.708647

p-value is  0.04195875

mean diff is  -0.04380032  and 95% CI is  -0.08502022 -0.00258042

For random-reader random-case (RRRC) analysis the observed value of the F-statistic is smaller than the critical value, the p-value is greater than 0.05 and the 95% confidence interval includes zero, all which tell us that the NH cannot be rejected at the 5% significance level. As one might expect, if one "freezes" reader or case variability, the difference becomes significant, whether viewed from the point of view of the F-statistic exceeding the critical value, the observed p-value being smaller than alpha or the 95% CI for the difference FOM not including zero. Also, compare these results to those obtained using mainDBMHBrief.R.

# Appendix 9.C: Text output file listing (deleted)

The contents of the traditional format output file "VanDykeOutput.txt" created by sourcing the code in mainDBMHBrief.R, can be readily viewed. Therefore the long listing has been deleted in this text. The output format is closely modeled on earlier versions of DBM-MRMC software from the University of Iowa ROC website.

# Appendix 9.D: Excel ANOVA Tables

Table 9.A.1: This shows the contents of the worksheet ANOVA in the Excel output file VanDykeOutput.xlsx.

| FOM variance components |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Var(R) | 0.00153 |  |  |  |  |  |
| Var(C) | 0.02725 |  |  |  |  |  |
| Var(T*R) | 0.00020 |  |  |  |  |  |
| Var(T*C) | 0.01198 |  |  |  |  |  |
| Var(R*C) | 0.01226 |  |  |  |  |  |
| Var(Error) | 0.03997 |  |  |  |  |  |
| TREATMENT X READER X CASE ANOVA |  |  |  |  |  |  |
| Source | SS | df | MS |  |  |  |
| T | 0.54676 | 1 | 0.54676 |  |  |  |
| R | 1.74931 | 4 | 0.43733 |  |  |  |
| C | 44.84630 | 113 | 0.39687 |  |  |  |
| TR | 0.25127 | 4 | 0.06282 |  |  |  |
| TC | 11.28283 | 113 | 0.09985 |  |  |  |
| RC | 29.15448 | 452 | 0.06450 |  |  |  |
| TRC | 18.06716 | 452 | 0.03997 |  |  |  |
| Total | 105.89811 | 1139 |  |  |  |  |
| READER X CASE ANOVA for each Trt |  |  |  |  |  |  |
| Source | df | Trt - 1 | Trt - 2 |  |  |  |
| R | 4.00000 | 0.35142 | 0.14872 |  |  |  |
| C | 113.00000 | 0.33629 | 0.16042 |  |  |  |
| RC | 452.00000 | 0.06044 | 0.04404 |  |  |  |
| TREATMENT X CASE ANOVAs (MS) for each reader, assuming fixed-reader analysis |  |  |  |  |  |  |
| Source | df | Rdr - 1 | Rdr - 2 | Rdr - 3 | Rdr - 4 | Rdr - 5 |
| T | 1 | 0.04527 | 0.12345 | 0.01821 | 0.03927 | 0.57184 |
| C | 113 | 0.12225 | 0.21275 | 0.13653 | 0.01729 | 0.16605 |

The variance components, specifically the ones with "treatment" in their names, are needed for sample size estimation, Chapter 11. The values listed under treatment x reader x case ANOVA allow one to test for interactions, Appendix 9.H.

# Appendix 9.E: Code for Validating DBMH analysis

Here is the code, file mainRejectRate.R, which performs NH testing. The corresponding cluster version, not listed, is in file mainRejectRateCluster.R.

### Appendix 9.E.1: Code Explanation

The simulator is implemented in RoeMetzZSamples.R sourced at line 5. The reader should examine this file and be convinced that it is a straightforward application of Eqn. (9.55) and Eqn. (9.56). Line 8 specifies the size of the datasets that will be generated for each trial, namely two treatments, five readers, 52 non-diseased and 50 diseased cases. The variance structure "LH1" is specified for the simulator (the notation follows that in the original Roe-Metz publication5). Line 9 obtains the variance structure using the function RoeMetzVarStr() which is part of the RoeMetzZSamples.R file. Line 10 prints out the values of the variance components, in the order , , , ,  and , followed by AUC and the name of the simulator. Note the usage of the unlist() function to assure everything prints out in one line, specifically, the printout is Var Str =  0.011, 0.011, 0.1, 0.1, 0.2, 0.6, 0.702, LH1. Commas have been inserted for readability. The first 6 values are the z-sample variance components, followed by AUC = 0.702 and the name of the simulator, LH1.

Line 11 defines the fixed effects in the simulator; AUC is converted to a separation parameter and tau22 is set to zero (by the zero multiplier, which assures the null hypothesis condition; if the multiplier is non-zero one is simulating the alternative hypothesis).

Line 12 defines isBinned = TRUE, which ensures that the data is binned into DesiredNumBins = 5 bins at line 24. Line 13 sets the simulator to conduct 2000 simulations and initializes the number of rejects (i.e., NH rejections) to zero. Lines 14 – 28 conduct the 2000 simulations inside a for-loop. Line 16 calls InitRandomSamples() to initialize all requisite samples in Eqn.  with samples from unit normal distributions. The multiplication by the correct variances is done in line 18-19. At this point one has the raw z-samples, denoted zSamplesRaw. Lines 20 -22 constructs an RJafroc dataset object using the simulated data, which permits ready application of functions in this package, a binning function at line 24 and the DBMH analysis function at line 26. Line 27 examines the p-value and if smaller than nomAlpha, increments the reject counter variable by one. After 2000 simulations one arrives at line 29, which calculates empirical alpha, called empAlpha. Line 30 calculates the 95% confidence interval and the rest contain print statements.

This code takes a while to execute. To see what is going on the reader should reduce the number of simulations S to 2 and step through the code by clicking on Run. A faster version of the code uses cluster computing, in file mainRejectRateCluster.R. This is harder to run in "slow-motion" to see what is going on.

# Appendix 9.F: Simulator for validating fixed-reader and fixed-case analyses

## Appendix 9.F.1: Fixed reader simulator

To simulate Z-samples for fixed-reader one sets= = 0 which does not affect the normalization requirement + +  +  = 1. The simulation model is:

.	.

## Appendix 9.F.2: Fixed case simulator

To generate data for fixed cases one sets == 0.   However, since + < 1, to restore normalization the variances  and  implicit in Eqn.  need to be inflated by dividing by + , resulting in new variances  and  defined by

.	.

Note that  +  = 1 ensuring that the simulator is properly normalized.  The simulation model is:

.	.

where

.	.

# Appendix 9.G: The meaning of pseudovalues: code explanation

The relevant file is mainMeaningPseudoValues.R. Line 16-17 extracts the number of treatments, readers and cases. Lines 23 – 24 begin two nested for-loops. Line 25 extracts the data for the selected treatment and reader. Line 26 extracts the two ratings arrays, zk1 and zk2, for non-diseased and diseased cases, respectively. Line 27 calculates the pseudovalues for the single-treatment single-reader dataset object rocData1T1R, using the Wilcoxon figure of merit. Line 28 removes unnecessary indices of unit length, and lines 29 and 30 extract the pseudovalues for non-diseased and diseased cases to two arrays, psdVlsNor and psdVlsAbn, respectively. The table() function used in line 32, 35 is quite useful. It orders the pseudovalues into unique values and computes the number of occurrences of each unique value. Line 33 prints this out for non-diseased case pseudovalues for while line 36 does the same for non-diseased cases. Lines 35 and 36 do the same for diseased case pseudovalues and ratings, respectively.

For this dataset there is one situation where the correspondences between the number of occurrences of pseudovalues and ratings appears to be violated. For treatment 2 and reader 5, Table 9.A.2, the number of unique pseudovalues and unique ratings are different. However, if one cumulates the two highest ratings counts, the direct correspondence is true. What actually happened in this situation is that the number of unique pseudovalues did not equal the number of ratings bins. For non-diseased cases the reader used only the 3 lowest ratings bins while for diseased cases all 5 ratings bins were used.

Table 9.A.2: Data for treatment 2 and reader 5 for diseased cases of the Van Dyke data set: ordered unique pseudovalues, row 1, the number of occurrences of each distinct pseudovalue, row 2, and corresponding values for ratings, rows 3 and 4. Note the direct correspondence between the ordering of the ratings and the ordering of the pseudovalues.

| Pseudovalues | -1.068 | 0.049 | 0.942 | 1.110 | NA | 0.919646 |
| --- | --- | --- | --- | --- | --- | --- |
| # occurrences | 1 | 4 | 10 | 30 | NA |  |
| Ratings | 1 | 2 | 3 | 4 | 5 | 4.2667 |
| # occurrences | 1 | 4 | 10 | 4 | 26 |  |

# Appendix 9.H: Testing for interactions

In Chapter 07 it was stated that between-reader variance could be treatment dependent, i.e., the readers interpretations might exhibit greater variability in one modality than in the other. For the between-reader variance to depend on treatment, then, with reference to Eqn. 9.7, the variance  must be significantly greater than zero. The actual value of  = 0.00020, is quite small (ANOVA table in Excel output file, Table 9.A.1). To determine if this value is significantly different from zero, consider the F-ratio  =  (also contained in cited ANOVA table). The observed value comes from an F-distribution with ndf = 4 and ddf = 452 (see ANOVA worksheet rows 14 and 17). Therefore, the p-value for testing for a treatment-reader effect is:

.

Calculating this in R, one has 1 - pf(0.06282/0.03997,4,452) = 0.1806607. As the p-value exceeds 0.05, the treatment-reader effect is not significant at alpha = 0.05, consistent with the rather small estimate for . A similar method can be used to test for a treatment-case effect. From the ANOVA output,  = 0.01198, which is much larger than .  The relevant F-ratio is  and the p-value is calculated using:

.

The treatment-case term is significantly different from zero. It is shown (§9.13) that the pseudovalues are interpretable as the FOMs of individual cases. With this interpretation in mind,  is the case-variance of the FOM for each treatment, averaged over treatments, and  indicates how much the case variance depends on treatment. It is possible for the same case-set to appear more homogenous in one treatment and more heterogeneous in the other treatment, as with this dataset, with treatment #1 having larger variability (0.33629 > 0.16042, ANOVA sheet row 23).

# References

1.	Satterthwaite FE. Synthesis of variance. Psychometrika. 1941 6(5):309–316.

2.	Satterthwaite FE. An Approximate Distribution of Estimates of Variance Components. Biometrics Bulletin. 1946;2(6):110–114.

3.	Larsen RJ, Marx ML. An Introduction to Mathematical Statistics and Its Applications. 3rd ed. Upper Saddle River, NJ: Prentice-Hall Inc; 2001.

4.	Field A. Discovering statistics using SPSS. Sage publications; 2009.

5.	Roe CA, Metz CE. Dorfman-Berbaum-Metz Method for Statistical Analysis of Multireader, Multimodality Receiver Operating Characteristic Data: Validation with Computer Simulation. Acad Radiol. 1997;4:298-303.

The reason for the slightly unequal numbers of cases is to be assured that array dimensioning is working as expected; it is the author's programming style.