Chapter 11: Sample size estimation for ROC studies

# Table of Contents

- Introduction

- Statistical power

- Sample size estimation

- Dependence of statistical power on estimates of model parameters

- Formulae for random-reader random-case (RRRC) sample size estimation

- Formulae for fixed-reader random-case (FRRC) sample size estimation

- Formulae for random-reader fixed-case (RRFC) sample size estimation

- Example 1

- Example 2

- Details of the sample size estimation process

- Cautionary notes: the Kraemer et al paper

- Prediction accuracy of sample size estimation method

- On the unit of effect-size: a proposal

- Discussion/Summary

- References

Online Supplementary Material

- Online Appendix 11.A: Sample size formulae using ORH approach

- Online Appendix 11.B: ORH fixed-readers random-case (FRRC)

- Online Appendix 11.C: ORH random-reader fixed-cases (RRFC)

- Online Appendix 11.D: Details on effect-size specification

# 11.1: Introduction

The question addressed in this chapter is "how many readers and cases", usually abbreviated to "sample-size", should one employ to conduct a "well-planned" ROC study. The reasons for the quotes around "well-planned" will become clear below. If cost were no concern, the reply would be: "as many readers and cases as one can get". There are other causes affecting sample-size, e.g., the data collection paradigm and analysis, however, this chapter is restricted to the MRMC ROC data collection paradigm, with data analyzed by the DBMH or the substantially equivalent ORH methods described in previous chapters. It turns out that provided one can specify comparable effect-sizes between different paradigms (i.e., in the same units, ideally AUCs under the ROC curves), the methods described in this chapter are applicable to other paradigms; see Chapter 18 for sample size estimation for FROC studies. For this reason it is important to understand the concepts of sample-size estimation in the simpler ROC context.

For simplicity and practicality, this chapter is restricted to analysis of two-treatment data (I = 2). The purpose of most imaging system assessment studies is to determine, for a given diagnostic task, whether radiologists perform better using a new treatment over the conventional treatment, and whether the difference is statistically significant. Therefore, the two-treatment case is the most common one encountered. While it is possible to extend the methods to more than two treatments, the extensions are not useful or clinically interesting.

Assume the figure of merit (FOM)  is chosen to be the area AUC under the ROC curve (empirical or fitted is immaterial as far as the formulae are concerned, so long as one sticks with a choice; however, the choice of fitted vs. empirical will affect statistical power; the dependence has not been studied). The statistical analysis determines the significance level of the study, i.e., the probability or p-value for incorrectly rejecting the null hypothesis (NH) that the two s are equal: , where the subscripts refer to the two treatments. If the p-value is smaller than a pre-specified , typically set at 5%, one rejects the NH and declares the treatments different at the  significance level. Statistical power is the probability of correctly rejecting the null hypothesis when the alternative hypothesis is true, Chapter 08.

The value of the true difference between the treatments, i.e., the true effect-size is, of course, unknown. If it were known, there would be no need to conduct an ROC study. One would simply adopt the treatment with the higher . Sample-size estimation involves making an educated guess regarding the true effect-size, called the anticipated ES, and denoted by . To quote1: "any calculation of power amounts to specification of the anticipated effect-size". Increasing the anticipated effect size will increase statistical power but may represent an unrealistic expectation of the true difference between the treatments, in the sense that it overestimates the ability of technology to achieve this much improvement. An unduly small might be clinically insignificant, besides requiring a very large sample-size to achieve sufficient power.

Statistical power depends on the magnitude of d divided by the standard deviation  of , i.e. . The sign is relevant as it determines whether the project is worth pursuing at all (see §11.8.4). The ratio is termed2 Cohen's D. When this signal-to-noise-ratio-like quantity is large, statistical power is large. Reader and case variability and data correlations determine . No matter how small the anticipated , as long as it is finite, then, using sufficient numbers of readers and cases can be made sufficiently small to achieve any desired statistical power. Of course, a very small effect-size may not be clinically significant. There is a key difference between statistical significance and clinical significance. An effect-size in AUC units could be so small, e.g., 0.001, as to be clinically insignificant, but by employing a sufficiently large sample size one could design a study to detect this small and clinically meaningless difference with high probability, i.e., high statistical power.

What determines clinical significance? A small effect-size, e.g., 0.01 AUC units, could be clinically significant if it applies to a large population, where the small benefit in detection rate is amplified by the number of patients benefiting from the new treatment. In contrast, for an "orphan" disease, i.e., one with very low prevalence, an effect-size of 0.05 might not be enough to justify the additional cost of the new treatment. The improvement might have to be 0.1 before it is worth it for a new treatment to be brought to market. One hates to monetize life and death issues, but there is no getting away from it, as cost/benefit issues determine clinical significance. The arbiters of clinical significance are engineers, imaging scientists, clinicians, epidemiologists, insurance companies and those who set government health care policies. The engineers and imaging scientists determine whether the effect-size the clinicians would like is feasible from technical and scientific viewpoints. The clinician determines, based on incidence of disease and other considerations, e.g., altruistic, malpractice, cost of the new device and insurance reimbursement, what effect-size is justifiable. Cohen has suggested that d values of 0.2, 0.5, and 0.8 be considered small, medium, and large, respectively, but he has also argued against their indiscriminate usage. However, after a study is completed, clinicians often find that an effect-size that biostatisticians label as small may, in certain circumstances, be clinically significant and an effect-size that they label as large may in other circumstances be clinically insignificant. Clearly, this is a complex issue. Some suggestions on choosing a clinically significant effect size are made in §11.12.

Having developed a new imaging modality the R&D team wishes to compare it to the existing standard with the short-term goal of making a submission to the FDA to allow them to perform pre-market testing of the device. The long-term goal is to commercialize the device. Assume the R&D team has optimized the device based on physical measurements, Chapter 01, perhaps supplemented with anecdotal feedback from clinicians based on a few images. Needed at this point is a pilot study. A pilot study, conducted with a relatively small and practical sample size, is intended to provide estimates of different sources of variability and correlations. It also provides an estimate of the effect-size, termed the observed effect-size, . Based on results from the pilot the sample-size tools described in this chapter permit estimation of the numbers of readers and cases that will reduce  sufficiently to achieve the desired power for the larger "pivotal" study. [A distinction could be made in the notation between observed and anticipated effect sizes, but it will be clear from the context. Later, it will be shown how one can make an educated guess about the anticipated effect size from an observed effect size.]

This chapter is concerned with multiple-reader MRMC studies that follow the fully crossed factorial design meaning that each reader interprets a common case-set in all treatments. Since the resulting pairings (i.e., correlations) tend to decrease  (since the variations occur in tandem, they tend to cancel out in the difference, see Chapter 09, Introduction, for Dr. Robert Wagner's sailboat analogy) it yields more statistical power compared to an unpaired design, and consequently this design is frequently used. Two sample-size estimation procedures for MRMC are the Hillis-Berbaum method3 and the Obuchowski-Rockette4-6 method. With recent work by Hillis, the two methods have been shown to be substantially equivalent.

This chapter will focus on the DBMH approach. Since it is based on a standard ANOVA model, it is easier to extend the NH testing procedure described in Chapter 09 to the alternative hypothesis, which is relevant for sample size estimation. Online Appendix 11.A shows how to translate the DBMH formulae to the ORH method7.

Given an effect-size and choosing this wisely is the most difficult part of the process, the method described in this chapter uses pseudovalue variance components estimated by the DBMH method to predict sample-sizes (i.e., different combinations of numbers of readers and cases) necessary to achieve a desired power.

The starting point is a brief recapitulation of the 2 x 2 table of hypothesis testing.

# 11.2: Statistical power

Table 11.1: Outcome of an ROC study: The two types of errors and correct decisions at the end of an ROC study. Note the close parallel to the fundamental 2 x 2 table of ROC analysis.

| Truth | Decision |  |
| --- | --- | --- |
|  | Fail to reject NH | Reject NH |
| NH is true |  |  |
| NH is false |  |  |

The concept of statistical power was introduced in Chapter 08 but it is worth repeating. There are two possible decisions following a test of a null hypothesis (NH): reject or fail to reject the NH. Each decision is associated with a probability on an erroneous conclusion, Table 11.1. If the NH is true and one rejects it, the probability of the ensuing Type-I error is . If the NH is false and one fails to reject it, the probability of the ensuing Type II- error is . Statistical power is the complement of , i.e.,

.

Thus, statistical power is defined as the probability of (correctly) rejecting the null hypothesis when the null hypothesis is false. Typically, one aims for  or less, i.e., a statistical power of 80% or more. Again, like alpha = 0.05, this is a convention, and more nuanced cost-benefit considerations may cause the researcher to adopt a different value in the study design.

# 11.3: Sample size estimation for random-reader random-cases (RRRC)

The procedure is illustrated using the DBMH significance testing method. The corresponding ORH based approach is described in Online Appendix 11.C. For convenience the DBMH model is repeated below with the case-set index suppressed:

.

As usual, the treatment effects  are subject to the constraint that they sum to zero. The observed effect size (a random variable) is defined by:

.

It is a realization of a random variable, so one has some leeway in the choice of anticipated effect size. In the significance-testing procedure described in Chapter 09 interest was in the distribution of the F-statistic when the NH is true. For sample size estimation, one needs to know the distribution of the statistic when the NH is false. It was shown that then the observed F-statistic Eqn. (9.35) is distributed as a non-central F-distribution  with non-centrality parameter :

.

The non-centrality parameter  was defined, Eqn. (9.34), by:

.

To minimize confusion, this equation has been rewritten here using the subscript Y to explicitly denote pseudo-value derived quantities (in Chapter 09 this subscript was suppressed; it is needed here because this chapter considers both DBMH and ORH methods).

To avoid a negative denominator, Hillis suggests the following modification:

.

This expression depends on three variance components, , the two terms are inseparable,  and . Eqn. (9.24) defines the ddf term appearing in Eqn. ; this quantity does not change between NH and AH:

.

The mean squares in this expression can be expressed in terms of the three variance-components appearing in Eqn. . Hillis and Berbaum3 have derived these expression and they will not be repeated here (Eqn. 4 in cited reference). RJafroc implements a function to calculate the mean squares, UtilMeanSquares(), which allows ddf to be calculated using Eqn. . The sample size functions in this package only need the three variance-components (the formula for ddf is implemented internally).

For two treatments, since the individual treatment effects must be the negatives of each other (because they sum to zero), it is easily shown that:

.

Therefore, for two treatments the numerator of the expression for  becomes . For three treatments, I = 3, assuming the reader-averaged FOMs are spaced at intervals of d, it is easily shown that the numerator of the expression for  becomes , which is 4 times the value for two treatments, meaning that power is expected to increase dramatically. For the rest of the chapter it is assumed that one is limited to two treatments.

Assuming no other similar studies have already been conducted with the treatments in question, the observed effect-size, although "merely an estimate", is the best information available at the end of the pilot study regarding the value of the true effect-size. From the two previous chapters one knows that the significance testing software will report not only the observed effect-size, but also a 95% confidence interval associate with it. It will be shown later how one can use this information to make an educated guess regarding the value of the anticipated effect-size.

# 11.4: Background on the F-distribution - Vignette 1

Navigate to https://dpc10ster.github.io/rjafroc-master/reference/index.html and select the article Chapter 11, Vignette 1, "Background on the F distribution". It is important to view the vignette simultaneously with reading this text.

Since it plays such a central role in sample size estimation, it is helpful to examine the behavior of the F-distribution. In the following ndf = numerator degrees of freedom and ddf = denominator degrees of freedom and ncp = non-centrality parameter (i.e., in Eqn. ).

The use of three functions is demonstrated.

- qf(p,ndf,ddf) is the quantile function of the F-distribution for specified values of ndf and ddf, i.e., the value x such that fraction p of the area under the F-distribution lies to the right of x.

- df(x,ndf,ddf,ncp) is the pdf or density function of the F-distribution for specified values of ndf, ddf and ncp.

- pf(x,ndf,ddf,ncp) is the probability (or cumulative) distribution function, of the F-distribution for specified values of ndf, ddf and ncp.

For each of four values of the non-centrality parameter - 0, 2, 5 and 10 - the vignette shows pdf plots for the following parameters values:

- ndf = 2, ddf = 10

- ndf = 2, ddf = 100

- ndf = 1, ddf = 100

Additionally, it lists the statistical power, i.e., the probability of exceeding the critical value of the F-distribution.

## 11.3.1: ndf = 2, ddf = 10

The vignette generates plots, not all shown here, and the following output.

critical value of x for rejecting NH is  4.102821

ndf =  2 , ddf =  10 , ncp =  0 , prob > fCrit =  0.05

ndf =  2 , ddf =  10 , ncp =  2 , prob > fCrit =  0.177584

ndf =  2 , ddf =  10 , ncp =  5 , prob > fCrit =  0.3876841

ndf =  2 , ddf =  10 , ncp =  10 , prob > fCrit =  0.6769776

| (A) | (B) |
| --- | --- |
| (C) | (D) |

Fig. 11.1: Plots of the F-distribution pdf for ndf = 2, ddf = 10 and different values of the non-centrality parameter ncp. A: ncp = 0; B: ncp = 2; C: ncp = 5; D: ncp = 10. For ncp = 0 the distribution is identical to the usual (i.e., central) F-distribution; as ncp increases the distribution shifts increasingly to the right, thereby making it more likely that a random sample will exceed a specified value, in other words, the NH is more likely to be rejected, and statistical power will increase.

- Plot (A) corresponds to ncp = 0, which yields the usual (aka central) F-distribution that was used in the previous two chapters. The integral under this distribution is unity (this is true for all plots in Fig. 11.1). The critical value for rejecting the NH denoted  , fCrit in the code, is that value of x such that the probability of exceeding x is , fCrit = 4.10. Notice the use of the quantile function qf() to determine this value, also the default value of ncp, namely zero, is used: specifically, line 3 does not pass a 4th argument to qf(). The decision rule for rejecting the NH uses the NH distribution of the F-statistic, i.e., the decision rule is to reject the NH if . As expected, prob > fCrit = 0.05 because this is how fCrit was defined.

- Plot (B) corresponds to ncp = 2. It is noticeably shifted to the right, thereby making it more likely that the observed value of the F-statistic will exceed the critical value determined for the NH distribution. This time prob > fCrit = 0.18, which is statistical power.

- Plot (C) corresponds to ncp = 5, prob > fCrit = 0.39.

- Plot (D) corresponds to ncp = 10, prob > fCrit is 0.68. The effect of the shift is obvious in plots (C) and (D). Considering a vertical line at x = 4.10, most of the distribution in plot (D), more than 67% to be precise, lies to the right of this line, so the NH is more than 67% likely to be rejected. Of course, what causes the shift is the non-centrality parameter. The larger that non-centrality parameter is, the greater the shift to the right.

Increasing ddf from 10 to 100 changes the output to the following (middle of vignette, where the corresponding plots – not shown here - are labeled E, F, G, H) and the following output.

## 11.3.2: ndf = 2, ddf = 100

critical value of x for rejecting NH is  3.087296

ndf =  2 , ddf =  100 , ncp =  0 , prob > fCrit =  0.05

ndf =  2 , ddf =  100 , ncp =  2 , prob > fCrit =  0.2199264

ndf =  2 , ddf =  100 , ncp =  5 , prob > fCrit =  0.4910802

ndf =  2 , ddf =  100 , ncp =  10 , prob > fCrit =  0.8029764

Increasing ddf by a factor of 10 decreases fCrit from 4.10 to 3.08, thereby increasing the probability that a sample from the non-central F-distribution will exceed the critical value. Therefore, power values increase compared to those shown for ddf = 10, but the effect is not as dramatic as that of increasing ncp.

## 11.3.3: ndf = 1, ddf = 100

critical value of x for rejecting NH is  3.936143

ndf =  1 , ddf =  100 , ncp =  0 , prob > fCrit =  0.05

ndf =  1 , ddf =  100 , ncp =  2 , prob > fCrit =  0.2883607

ndf =  1 , ddf =  100 , ncp =  5 , prob > fCrit =  0.6004962

ndf =  1 , ddf =  100 , ncp =  10 , prob > fCrit =  0.8793619

Last, illustrated above (bottom of vignette, where the corresponding plots – not shown here - are labeled I, J, K, L) is the increase in power with fewer treatments (ndf = I -1). At first glance this may seem counterintuitive, as fCrit has gone up, but can be explained by the differing shapes of the two distributions: the pdf is broader for ndf = 1 as compared to ndf = 2 (compare plot L to H).

# 11.4: Dependence of statistical power on estimates of model parameters

Examination of the expression for , Eqn. , shows that statistical power increases if:

- The numerator is large. This occurs if: (a) the anticipated effect-size d is large. Since effect-size enters as the square, Eqn. , it is has a particularly strong effect; (b) If J x K is large. Both of these results should be obvious, as a large effect size and a large sample size result in increased probability of rejecting the NH.

- The denominator is small. The first term in the denominator is . The two terms cannot be separated. This is the residual variability of the jackknife pseudovalues. It makes sense that the smaller the variability, the larger is the non-centrality parameter and the statistical power.

- The next term in the denominator is , the treatment-reader variance component multiplied by the total number of cases. The reader variance  has no effect on statistical power, because it has an equal effect on both treatments and cancels in the difference. Rather, the treatment-reader component  contributes "noise" confounding the estimate of the effect-size.

- The variance components estimated by the ANOVA procedure are realizations of random variables and as such subject to noise (there exists a beast such as variance of a variance). The presence of the K term, usually large, can amplify the effect of noise in the estimate of , making the sample size estimation procedure less accurate.

- The final term in the denominator is . The variance  has no impact on statistical power, as it cancels in the difference. The treatment-case variance component introduces "noise" into the estimate of the effect size, thereby decreasing power. Since it is multiplied by J, and typically , the error amplification effect is not as bad as with the treatment-reader variance component.

# 11.5: Formulae for random-reader random-case (RRRC) sample size estimation

Having performed a pilot study and planning to perform a pivotal study, sample size estimation follows the following procedure, which assumes that both reader and case are treated as random factors. Different formulae, described later, apply when either reader or case is treated as a fixed factor.

- Perform DBMH analysis on the pilot data. This yields the observed effect size as well as estimates of all relevant variance components and mean squares appearing in Eqn.  and Eqn. .

- This is the difficult but critical part: make an educated guess regarding the effect-size, , that one is interested in "detecting" (i.e., hoping to reject the NH with probability ). The author prefers the term "anticipated" effect-size to "true" effect-size (the latter implies knowledge of the true difference between the modalities which would obviate the need for a pivotal study). Two scenarios are considered below. In the first scenario, the effect-size is assumed equal to that observed in the pilot study, i.e., . In the second, so-called best-case scenario, one assumes that the anticipate value of d is the observed value plus two-sigma of the confidence interval, in the correct direction, of course, i.e., . Here  is one-fourth the width of the 95% confidence interval for . Anticipating more than  greater than the observed effect-size would be overly optimistic. The width of the CI implies that chances are less than 2.5% that the anticipated value is at or beyond the overly optimistic value. These points will become clearer when example datasets are analyzed below.

- Calculate statistical power using the distribution implied by Eqn. , to calculate the probability that a random value of the relevant F-statistic will exceed the critical value, as in §11.3.2.

- If power is below the desired or "target" power, one tries successively larger value of J and / or K until the target power is reached.

# 11.6: Formulae for fixed-reader random-case (FRRC) sample size estimation

It was shown in §9.8.2 that for fixed-reader analysis the non-centrality parameter is defined by:

.

The sampling distribution of the F-statistic under the AH is:

.

# 11.7: Formulae for random-reader fixed-case (RRFC) sample size estimation

It is shown in §9.9.1 that for fixed-case analysis the non-centrality parameter is defined by:

.

Under the AH, the test statistic is distributed as a non-central F-distribution as follows:

.

# 11.8: Illustration of formulae using vignettes

It is instructive to see the formulae implemented in software.

## 11.8.1: Vignette 2

Shown is the "open" implementation, where by "open" I mean it shows "all the nitty-gritty details" of the steps. The relevant vignette is Chapter 11, Vignette 2: "ROC-DBMH sample size from first principles". This illustrates the DBMH method using the Van Dyke dataset8 contained in VanDyke.lrc. In this example, the observed effect size in the pilot study is used as the anticipated effect size for the pivotal study – this is generally not a good idea as discussed later under "Cautionary notes". Illustrated in this vignette are RRRC, FRRC and RRFC analyses. Intermediate quantities such as the critical value of the F-statistic, ddf and ncp are shown (this is the so-called "nitty-gritty"). The reader should confirm that the code does in fact implement the relevant formulae.

## 11.8.2: Vignette 3

Chapter 11, Vignette 3: " RJafroc functions for ROC sample size estimation using DBMH method" illustrates the usage of three RJafroc functions related to sample size estimation for ROC studies (sample size estimation for FROC studies is dealt with in Chapter 18). Here the details are hidden from the user, making for relatively easy to use functions. These functions are (the arguments are not shown; they can be viewed at https://dpc10ster.github.io/rjafroc-master/reference/index.html):

- SsPowerGivenJK()

Statistical power for specified numbers of readers and cases.

- SsPowerTable()

Generate a power table, i.e., combinations of numbers of readers and cases yielding the desired power.

- SsSampleSizeKGivenJ

Number of cases, to achieve desired power, for specified number of readers.

## 11.8.3: Vignette 4

The use of the above functions, but this time using the ORH method to estimate the variance components, is illustrated in Chapter 11, Vignette 4: RJafroc functions for ROC sample size estimation using ORH method. The reader should confirm that these give the same results as the corresponding ones obtained using the DBMH method. When the figure of merit is the empirical AUC, the two methods can be shown to be identical.

# 11.9: Choosing the effect size: Example 1

rm(list = ls()) #mainSsDbmh.R

library(RJafroc)

fileName <- "VanDyke.lrc"

#fileName <- "Franken1.lrc"

OptimisticScenario <-  FALSE;alpha <- 0.05

rocData <- DfReadDataFile(fileName, format = "MRMC")

retDbm <- SignificanceTesting(dataset = rocData, fom = "Wilcoxon", method = "DBMH")

varYTR <- retDbm$varComp$varComp[3];varYTC <- retDbm$varComp$varComp[4];varYEps <- retDbm$varComp$varComp[6]

effectSize <- retDbm$ciDiffTrtRRRC$Estimate

cat("effect size = ", effectSize, "\n");effectSize <- abs(effectSize)

sigma <- (retDbm$ciDiffTrtRRRC$`CI Upper`-retDbm$ciDiffTrtRRRC$`CI Lower`)/4

if (OptimisticScenario == TRUE) {

effectSize <- effectSize + 2*sigma

}

cat("p-value = ", retDbm$pRRRC, ", anticipated effectSize = ",

effectSize, ", CI Lower =", retDbm$ciDiffTrtRRRC$`CI Lower`,

", CI Upper =", retDbm$ciDiffTrtRRRC$`CI Upper`, "\n")

powTab <- SsPowerTable(alpha = alpha, effectSize = effectSize, desiredPower = 0.8,

method = "DBMH", option = "ALL", varYTR, varYTC, varYEps)

print(powTab)

At line 6 the flag OptimisiticScenario is set to FALSE, which means the anticipated effect-size, i.e., the value entering the formulae stated above is set equal to the observed effect-size. If this flag were set to TRUE, the anticipated effect-size would be two-sigma greater than the observed value in the "correct" direction; this is implemented at line 14. Line 12 calculates sigma as one-fourth the width of the 95% confidence interval. Line 20-21 uses the function SsPowerTable() to generate the power table and the last line prints it. Notice option = "ALL" requesting RRRC, FRRC and RRFC analyses. Also effectSize = effectSize is not a tautology; on the left hand effectSize is a case-sensitive option understood by the function that is being called, while the right hand effectSize is the value defined at line 10 and 14. The option = "ALL", requests all three analyses (RRRC, FRRC and RRFC). Source the file mainSsDbmh.R to get the following output:

## 11.8.4: Code output (partial)

> source('~/book2/03 B Statistics of ROC analysis/B11 Sample Size/software/mainSsOrh.R')

File name =  VanDyke.lrc

alpha =  0.05

p-value =  0.05166569 , effectSize =  -0.04380032 , CI Lower = -0.0879595 , CI Upper = 0.0003588544

$powerTableRRRC

numReaders numCases power

2          3    >2000  <NA>

3          4     1089   0.8

4          5      344 0.801

5          6      251 0.801

6          7      211 0.801

7          8      188 0.801

8          9      173 0.801

9         10      163 0.802

...

$powerTableFRRF

numReaders numCases power

2          3      209 0.800

3          4      182 0.801

4          5      166 0.802

5          6      155 0.801

6          7      147 0.801

7          8      141 0.801

8          9      137 0.802

9         10      133 0.801

...

$powerTableRRFC

numReaders numCases power

2          3    >2000  <NA>

3          4    >2000  <NA>

4          5      289   0.8

5          6      151 0.801

6          7      102   0.8

7          8       78 0.803

8          9       63 0.804

9         10       53 0.805

...

The "power table" lists the different combinations of numbers of readers and cases that yield target power of 80% at an alpha of 5%. The listing is in three groups, corresponding to RRRC, FRRC and RRFC generalizations. For example, with four readers, RRRC analysis, 1089 cases are needed, clearly impractical, with five one needs 344 cases with six one needs 251 cases, etc. Based on this table, one might reasonably select 6 readers and about 250 cases. [With less than four readers the program predicts greater than 2000 cases, and besides one can hardly claim random-reader generalization with less than four readers.]

The sample-size values shown above are summarized in Table 11.2, which lists results for both datasets (VD = VanDyke, FR = Franken), both scenarios and all three generalizations. The numbers in the shaded part to the table are the numbers of cases needed to achieve 80% power for the indicated number of readers J. Estimates are shown for three cases: flag OptimisiticScenario set to FALSE, TRUE and effect size arbitrarily set to 0.05 for the Van Dyke dataset and 0.03 for the Franken dataset.

Table 11.2: The numbers in the shaded part to the table are the numbers of cases K needed to achieve 80% power for the indicated number of readers J. [VD = VanDyke, FR = Franken dataset; d = anticipated effect-size; Gen. = generalization; RRRC = random-reader random-case, FRRC = fixed-reader random-case, RRFC = random-reader fixed-case.]

| Data | Optimisitic Scenario | d | Gen. | # readers J |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| VD | FALSE | -0.0438 | RRRC | 1089 | 344 | 251 | 211 | 188 | 173 | 163 |
|  |  |  | FRRC | 182 | 166 | 155 | 147 | 141 | 137 | 133 |
|  |  |  | RRFC | >2000 | 289 | 151 | 102 | 78 | 63 | 53 |
|  | TRUE | -0.0659 | RRRC | 62 | 50 | 44 | 40 | 38 | 36 | 35 |
|  |  |  | FRRC | 47 | 43 | 40 | 38 | 37 | 36 | 35 |
|  |  |  | RRFC | 62 | 35 | 24 | <20 | <20 | <20 | <20 |
|  | NA |  | RRRC | 361 | 213 | 170 | 148 | 134 | 125 | 119 |
|  |  | -0.05 | FRRC | 140 | 128 | 119 | 113 | 109 | 106 | 103 |
|  |  |  | RRFC | 529 | 166 | 99 | 70 | 55 | 45 | 38 |
| FR | FALSE | 0.0109 | RRRC | >2000 | >2000 | >2000 | >2000 | 1666 | 1419 | 1237 |
|  |  |  | FRRC | >2000 | 1958 | 1632 | 1400 | 1225 | 1089 | 980 |
|  |  |  | RRFC | >2000 | >2000 | >2000 | >2000 | 1666 | 1419 | 1237 |
|  | TRUE | 0.0188 | RRRC | 926 | 579 | 421 | 332 | 274 | 233 | 203 |
|  |  |  | FRRC | 404 | 323 | 270 | 232 | 203 | 181 | 163 |
|  |  |  | RRFC | 926 | 579 | 421 | 332 | 274 | 233 | 203 |
|  | NA | +0.03 | RRRC | 739 | 462 | 336 | 265 | 219 | 186 | 162 |
|  |  |  | FRRC | 323 | 259 | 216 | 185 | 162 | 145 | 130 |
|  |  |  | RRFC | 739 | 462 | 336 | 265 | 219 | 186 | 162 |

It is clear from the table that the Van Dyke dataset is the "easier" one in the sense that fewer resources are needed to plan a study with 80% power.

First question: does one even need to perform a pivotal study? If the pilot study returns a significant difference, one has rejected the NH and that is all there is to it. There is no need to perform the pivotal study, unless one "tweaks" the new treatment and/or casts a wider sampling net to make a stronger argument, perhaps to the FDA, that the treatments are indeed generalizable, and that the difference is in the right direction (new treatment FOM > conventional treatment FOM). If a significant difference is observed in the opposite direction (e.g., new treatment FOM < conventional treatment FOM) one cannot justify a pivotal study with an expected effect-size in the opposite direction; see example below. Since the Van Dyke pilot study came close to rejecting the NH and the observed effect size, see below, is not too small, a pivotal study is justified.

Second question: what results did the pilot study return? The results of the analysis are contained in the variable retDbm (line 8) and the FOMs are contained in retDbm$fomArray. Use the code below to list the FOMs, the F-statistic, the ddf and the p-value (notice how RStudio gives helpful hints as one starts typing in a variable name; also, available list members are shown when one enters $ following a list variable).

## 11.8.5: Code snippet

> retDbm$fomArray

Rdr - 0   Rdr - 1   Rdr - 2   Rdr - 3   Rdr - 4

Trt - 0 0.9196457 0.8587762 0.9038647 0.9731079 0.8297907

Trt - 1 0.9478261 0.9053140 0.9217391 0.9993559 0.9299517

Browse[2]> retDbm$fRRRC;retDbm$ddfRRRC;retDbm$pRRRC

[1] 4.456319

[1] 15.25967

[1] 0.05166569

> retDbm$ddfRRRC;retDbm$ddfFRRC;retDbm$ddfRRFC

[1] 15.25967

[1] 113

[1] 4

> retDbm$pRRRC;retDbm$pFRRC;retDbm$pRRFC

[1] 0.05166569

[1] 0.02103497

[1] 0.04195875

For RRRC analysis the study did not reach significance, p-value = 0.0517. The observed F-statistic is 4.46 with degrees of freedom ndf = 1 (this is always # treatments minus 1 and is not printed), ddf = 15.3 and effectSize = -0.0438.

This is the difference (treatment 1 minus treatment 2) between the reader-averaged FOMs in the two treatments. Lacking any other information, the observed effect-size is the best estimate of the effect-size to be anticipated. Assuming treatment 2 is the new treatment, and because the effect size is negative the difference is going the right way. Because the pilot study did not reach significance, there is reason to conduct a pivotal study. A reasonable choice would be six readers and 251 cases, §11.8.4.

To print the upper and lower 95% confidence intervals, type in retOrh$ciDiffTrtRRRC$ and select the appropriate choice (in the code snippet below it is not necessary to type in the single quote etc.; just use RStudio's prompting abilities to the fullest) and repeat with the other appropriate choice. Observe that these values are centered on the effectSize value printed above:

## 11.8.6: Code snippet

Browse[2]> retOrh$ciDiffTrtRRRC$`CI Lower`;retOrh$ciDiffTrtRRRC$`CI Upper`

[1] -0.0879595

[1] 0.0003588544

The observed effect-size is a realization of a random variable. The mean of the confidence interval (CI) is -0.044, and one could use this as a reasonable anticipated value, and calculate sample size as was done above with the OptimisticScenario flag set to FALSE (the sample size does not depend on the sign of the effect-size, but the decision to perform a pivotal study most definitely does). CI's generated like this, with independent sets of data, are expected to encompass the true value with 95% probability. The lower end (greatest magnitude of the difference) of the confidence interval is -0.088, which is the optimistic estimate of the anticipated effect-size obtained with the OptimisticScenario flag set to TRUE, and yields small sample sizes, Table 11.2. For example, the number of cases is 50 for five readers and RRRC generalization. With this low number one would be justified in anticipating a smaller effect-size, e.g., -0.05. After all, -0.05 is only slightly greater in magnitude than the observer effect size -0.044. A 2004 publication3 used this effect size to illustrate the methodology for the Van Dyke dataset. The results for this effect size are in the Van Dyke section of the table, in the row marked NA, because the OptimisticScenario flag is irrelevant when effect size is overridden.

# 11.9: Choosing the effect size: Example 2

In the second example the Franken dataset9 is considered as the pilot study. Modality 1 is digital imaging of neonatal chest anatomy and modality 2 is conventional analog imaging. Reverse the commenting on lines 4 and 5 of file mainSsOrh.R to analyze this dataset. Sourcing it with OptimisticScenario flag set to FALSE yields the output summarized in Table 11.2 in the row labeled FALSE. The large numbers of cases indicate that, based on cost considerations, this dataset may not justify a pivotal study. Why are so many cases needed? Either the effect-size is too small and / or the variances are too large. Actually, both are true. The observed effect-size (0.011) is a factor of four smaller than that for the Van Dyke data. The Franken dataset was acquired in the early days of digital imaging technology, and scores of papers were published using ROC analysis to determine the pixel size requirements to match conventional analog imaging. Nowadays, digital technology has matured and practically replaced analog in all areas of medical imaging in the US. Results for OptimisticScenario flag set to TRUE are also summarized in Table 11.2. This corresponds to d = 0.0188. Also shown are results for an anticipated effect-size of +0.03, which is a factor of 1.6 larger than the best-case scenario, and maybe too optimistic considering then existing digital technology. Even with this effect size, the Franken dataset is a more difficult one, in the sense of requiring greater resources to achieve 80% power. All of the Franken dataset numbers in the shaded part of Table 11.2 are larger than the corresponding Van Dyke values.

## 11.9.1: Changing default alpha and power

Look at the help page for SsPowerTable(). [To find the help page for any R package, see Chapter 03, Online Appendix 3.E and Online Appendix 3.F]. Under Usage one sees:

SsPowerTable(alpha = 0.05, effectSize = 0.05, desiredPower = 0.8,

method = "DBMH", option = "ALL", ...)

The default effect-size (0.05) is already being overridden at line 20 by effectSize = effectSize. To override the default alpha to 0.01 change line six of file mainSsDbmh.R to 0.01. With the Van Dyke dataset selected and OptimisticScenario flag set to FALSE, source the file, yielding:

## 11.9.2: Code Output (partial)

alpha =  0.01

effect size =  -0.04380032

p-value =  0.05166569 , anticipated effectSize =  0.04380032 , CI Lower = -0.0879595 , CI Upper = 0.0003588544

$powerTableRRRC

numReaders numCases power

1           3    >2000  <NA>

2           4    >2000  <NA>

3           5    >2000  <NA>

4           6      962   0.8

5           7      482   0.8

6           8      371 0.801

7           9      317   0.8

8          10      285   0.8

If one wishes to control the probability of a Type I error to less than 1%, the price paid is a greatly increased sample size. For six readers, instead of 251 cases at alpha = 5%, one needs 962 cases at alpha = 1%. The reason for this should be clear from Chapter 08. A smaller alpha implies a larger critical value for the F-statistic. A similar procedure is used to override the default power.

# 11.10: Cautionary notes: the Kraemer et al paper

A paper by Kraemer et al10 titled "Caution regarding the use of pilot studies to guide power calculations for study proposals" is informative. Everything hinges on the choice of effect-size. Once it is specified, everything else follows from the sample size formulae (with due specification of the two types of errors and the desired generalization). There are three strategies for dealing with the quandary of how to choose the effect size.

- In the first strategy, a convenient sample size is set, and whatever statistical-power results is accepted. This is common in this field, and the author is aware of comments by acknowledged experts to the effect: "use 6 readers and about 100 cases, equally split between non-diseased and diseased and one should be fine". Indeed, many studies cluster around these values (even smaller sample-sizes have been used in some publications).

- In the second strategy, researchers set the effect-size based on their experience with other ROC studies, sometimes without consideration of the specific clinical context of the proposed study.

- In the third strategy, the one forming the basis of this chapter, the researchers propose (or institutional review boards, IRBs, insist) that a small pilot study be conducted to estimate the effect-size. After the pilot study is completed the researchers (or the IRB) makes a post-hoc (i.e., after the fact) decision whether the observed effect-size is clinically significant. If it does not meet the criterion, the study would not be proposed at all or, if proposed, would not be funded (i.e., aborted). On the other hand, if the pilot study observed effect-size is clinically significant, power calculations for the main study are conducted based on it.

Kraemer et al10 show that the most likely outcomes from the third strategy are: (1) studies worth performing are sometimes aborted and (2) studies that are not aborted are sometimes underpowered. This is a sobering thought and is something the researcher should keep in mind to guard against prematurely giving up on a new idea or, at the other extreme, being excessively optimistic about a new idea. The Kraemer et al paper is actually an interesting read and highly recommended.

# 11.11: Prediction accuracy of sample size estimation method

The observed power is a realization of a random variable: there is no guarantee that using the predicted sample size will actually achieve 80% power. Checking prediction accuracy of a sample size estimating method requires simulations similar to those used to test NH validity, §9.12. [If one studies the code used for checking NH validity in Chapter 09, in file mainRejectRate.R at line 11 one sees tau22 <- 0.0*mu2. The zero multiplier specifies the NH condition. If a non-zero multiplier is used, then the AH condition is true and the observed NH rejection fraction, over 2000 simulations, is an estimate of true power. The reason for using a multiplier, rather than an additive term, will become clear below.]

The author has published a paper where the prediction accuracy of the sample size estimation method described in this chapter was assessed11. Two thousand NH pilot datasets were generated using two Roe and Metz ratings simulators12 with complementary characteristics. Each data set consisted of 5 readers interpreting 50 normal and 50 abnormal cases in two modalities under the NH condition. The base-line NH area under the ROC curve was AUC = 0.855. Each simulated pilot dataset was analyzed by DBMH to get the necessary pseudovalue variance components. Then the sample size estimation procedure (Online Appendix 11.A presents sample size formulae using ORH FOM variance components as inputs) was applied to predict the number of cases  (i = 1, 2, …, 2000), assuming 10 readers in the pivotal study, to achieve 80% power for an anticipated effect-size of 0.06 in AUC units. For each predicted , one could conduct 2000 additional simulations under AH conditions to estimate true power, but in practice a simple interpolation procedure was used to reduce the computational burden11. (A - B) shows typical interpolation curves for the three generalization conditions for the two simulators studied, one with high reader variance and low case variance (A) and the other with the opposite characteristics (B).

| (A) | (B) |
| --- | --- |

Fig. 11.2: Figure (A) is the interpolation curves for the high reader variance low case variance simulator. Random-all (i.e., RRRC) yields the lowest curve implying more cases needed for 80% power.  The random-case (FRRC) curve is considerably upward-left shifted relative to RRRC, the effect of "freezing" the large reader variability, while the RRFC curve is less upward-left shifted. FRRC generalization will require hewer cases relative to RRFC generalization to achieve a desired power. (B) This figure, similar to (A), is for the low reader variance, high case variance simulator. The FRRC curve is slightly upward-left shifted relative to RRRC, implying a small reduction in number of cases for 80% power, but the RRFC curve is considerably upward-left shifted, implying a large reduction in needed number of cases for desired power. RANDOM-ALL = RRRC; RANDOM-CASES = FRRC; RANDOM-READERS = RRFC. Reproduced with permission from Ref. 11.

These plots illustrate several points. An obvious one is that power increases with the number of cases. A less obvious point is that for a given number of cases RRRC generalization yields the least power (solid lines). Focusing on Fig. 11.2 (A) a subtle point is that for the high reader variability simulator, freezing reader variability (dashed line) yields the greatest increase in power as compared to freezing case-variability (dotted line). Because Fig. 11.2 (B) is a low reader-variability high case-variability simulator, the opposite trends are observed. A final subtle point: by reader and case variability, one means the treatment-reader  and treatment-case  pseudovalue variance components, respectively, and of course, the term , which involves both reader and treatment variability. The treatment-independent terms,  and case variability , do not affect sample size, as these contribute constant shifts in performances between the two treatments.

Sometimes the sample size method failed to find a number of cases value below 2000, referred to as "clipping". In practice a pilot study yielding >2000 cases would need to be reanalyzed some other way, e.g., changing the method of estimating AUC, increasing the effect-size, or number of readers, to avoid clipping. Using the interpolation procedure, e.g., Fig. 11.2, the number of cases  and , were determined that corresponded to 0.75 and 0.90 powers. The fraction of the 2000 pilot study simulations where  was included in the range and was defined as , the quality or prediction-accuracy of the SS method (the rationale for the asymmetric interval around 80% is that a slight overestimate is desirable to an underestimate), i.e., = . The final HB prediction,, was defined as the median ofover the trials whereand the corresponding power was determined from the appropriate interpolation function.

| (A) | (B) |
| --- | --- |

Fig. 11.3: This figure shows the normalized histogram of  (i = 1, 2, ..., 2000) for the LH (low reader variability high case variability) simulator under the random-all condition.  Each value of i corresponds to an independent pilot data set.  The area under the histogram between the lines labeled  and  is the prediction-accuracy = 38% and  = 225 and  = 162.  The peak at 2000 cases representing the clipped predictions contributed 13% to the area.  (B) Similar to (A) except this applies to the HL simulator (high reader variability low case variability) under the random-all condition: = 26%,  = 159 and  = 190.  Note the large reduction in the prediction-accuracy performance index.  The area contributed by the peak at 2000 cases is 39%. Reproduced with permission from Ref. 11.

(A-B) show normalized histograms of  (i = 1, 2, ..., 2000) for the low (A) and high (B) reader variability simulators, respectively, under the random-all condition.  In Fig. 11.3 (A) the area under the histogram between the lines labeled  and is the prediction-accuracy = 38% and  = 225 (the Hillis-Berbaum prediction for required number of cases) and  = 162 (true value for required number of cases). An overestimate of the required number of cases is not necessarily a bad thing. The peak at 2000 cases representing the clipped predictions, contributes 13% to the area. In (B) = 26%,  = 159 and  = 190, so the HB method is underestimating the number of cases. The area contributed by the peak at 2000 cases is 39%. Prediction-accuracy was generally higher under low reader variability conditions than under high reader variability condition.

Appendix B of the referenced paper has a discussion of why the method is not as good for large reader variance. The reason has to do with the higher variability of the modality-reader variance component, Table 1, ibid. Since the number of cases, which could be in the hundreds, multiplies it, the net effect on variability is amplified. An overestimate of tends to decrease the power and the HB-method compensates by increasing the number of cases. A sufficiently large overestimate leads to clipping.

# 11.12: On the unit for effect-size: a proposal

Effect-size in ROC analysis is almost invariably specified in AUC units. Typically, 0.05 is considered a small to moderate effect-size13. Because AUC is restricted to the range (0,1), this creates the problem that the meaning of an effect-size depends on the baseline NH value of AUC to which it is added. An effect-size of 0.05 might qualify as a small effect-size if the baseline value is 0.7, since the improvement is from 0.70 to 0.75. On the other hand, if the baseline value is 0.95, then the same effect-size implies the new treatment has AUC = 1.00, in other words, performance is perfect, implying an infinite effect-size, see Fig. 11.4.

In the author's judgment, it makes more sense to specify effect-size in  units, where baseline or NH  is defined as follows (is not to be confused with Cohen's D):

.

This equation is derived from Eqn. (3.28), i.e., one simply imposes the equal variance binormal model, and calculates the separation parameter yielding the observed based line AUC. One can now specify small, medium and large effect-sizes, on the  scale, as multiples of the baseline  value, say 0.2, 0.4 and 0.6. For example, with the multiplier equal to 0.2, the AH  would be 1.2 x .

Fig. 11.4 (A) shows, for different baseline or NH values of AUC, the effect-size on the  multiple scale, U-shaped black curve, is for a fixed effect-size = 0.05 on the AUC scale, the gray line is for the  multiplier scale. As expected, near about 0.95, the effect-size on the  scale approaches a very large value, as one is demanding an improvement from 0.95 to 1.00. The y-axis of the plot, labeled "DpMultipler" is the factor by which  would have to increase to give the desired AUC effect-size of 0.05. For example, a value of one means the separation parameter would need to double to achieve the desired AUC effect-size. The increase in the black curve at the low end, near AUC = 0.5, is because there the baseline  approaches 0, so any increase in separation would be infinite multiplier compared to the baseline. The code generating these figures is in Online Appendix 11.D.

Fig. 11.4 (B) shows the effect-size on the AUC scale, black curve, for a fixed effect-size expressed as a multiplier equal to 0.2, the grey straight line, as a function of baseline AUC. This could be viewed as the other side of the story told by Fig. 11.4 (A). If one keeps the multiplier equal to 0.2, then effect-size in AUC units must decrease at the two ends of the plot where the corresponding Fig. 11.4 (A) plot increases. Note that with this way of specifying the effect-size the AH AUC is always in the valid range. Because in the near flat region near the center, the effect-size in AUC units is close to 0.04, the author proposes a multiplier equal to 0.2 as a small effect-size. Fig. 11.4 (C) shows the corresponding plot for multiplier equal to 0.4, which the author proposes as a medium effect-size, since the maximum AUC effect-size is about 0.08. Finally, Fig. 11.4 (D) shows the plot for multiplier equal to 0.6, which the author proposes as a large effect-size, since the maximum AUC effect-size is about 0.12.

| (A) | (B) |
| --- | --- |
| (C) | (D) |

Fig. 11.4: Different ways of specifying effect-size. Plot (A) corresponds to the conventional way of using a fixed difference in AUC, which implies infinite relative increase in separation parameter at 0.5 and 1. Plots (B – D) correspond to the preferred way of specifying effect-size, namely as a multiple of the baseline value, which avoids the infinities. Plot (B) a small effect-size, expressed as a constant multiple, 0.2, of . Plot (C) represents a medium effect-size as the multiplier is 0.4. Plot (D) represents a large effect-size, as the multiplier is 0.6. The code generating these figures is in Online Appendix 11.D.

The reader may wonder why the  based effect-size is specified as a multiplier instead of as an additive effect. Consider the task of detecting small blood vessel stenosis in cranial x-rays. The diagnostic medical physicist is probably familiar with "cross-table" views used to acquire these images in patients with cranial aneurisms or stroke. [The author constructed a digital subtraction angiography (DSA) apparatus for this very purpose14.] Just prior to the imaging, acquired rapidly at about one per second in those days, the patient is injected with iodine contrast directly into an internal carotid artery. The cranial vessels light up and one uses DSA to subtract the non-vessel background and thereby improve visualization of the blood vessels. So, this is the key point relevant to this book: the SNR of vessels is proportional to the iodine concentration and proportional (roughly) to the square of the diameter of the vessel, as the latter determines the volume of iodine in unit length of the vessel. A completely blocked vessel, where iodine cannot penetrate, will have SNR = 0 (i.e., no iodine signal) and a large vessel with greater volume of iodine per unit length will have greater SNR. This suggest the following model for :

.

Here is the baseline NH perceptual SNR of the vessel and  is effect-size expressed as a multiplier. If the baseline value is zero, the vessel has zero diameter and no amount of iodine can bring it out. On the other hand, if baseline is large, the vessel has finite diameter and effect of the iodine will be proportionately larger.

One last example before moving on: the author implemented a method called computer analysis of mammography phantom images (CAMPI) referred to in Chapter 01, as a way of quantifying mammography phantom images. Fig. 11.5 is an example from one15 of the published CAMPI studies15-20.

Fig. 11.5: (A): shows a plot of individual speck SNRs as measured on a test image vs. averaged individual speck SNRs measured on the insert images. There are 18 points in all, corresponding to the 18 specks in the first 3-microcalcification groups in the ACR phantom. (B): A similar plot to (a) except that an insert image has been plotted along the y-axis; as expected, its slope is close to unity.

The target objects (i.e., meant to be detected) are three speck groups, each containing 6 specks, meant to simulate microcalcifications, see Fig. 1.1 (A). A brief background on CAMPI16-20 is necessary. There are two types of images involved in CAMPI, insert images and test images. Insert images are contact-radiographs of a thin wax plate embedded in which are the target objects; a number of insert images are obtained under low kVp conditions to yield high quality images of the specks (low kVp increases subject contrast and the thin plate means that there is minimal scatter degradation of the images). The test images, on the other hand, are obtained under normal conditions with the insert inside a 4.5 cm thick Lucite block. Higher kVp is needed to penetrate the thicker phantom, and both contrast and scatter degrade the image compared to images of the insert. All images were digitized using a Lumisys Model LS-100 digitizer (Lumisys Inc., Sunnyvale, CA), with a pixel size of 50µm x 50µm and a gray level resolution of 12 bits. All insert images were digitally aligned and averaged to further reduce noise.

Fig. 11.5 (a) shows test image SNR (signal to noise ratio) for each speck on a test image plotted against corresponding averaged insert image SNRs. It follows a straight line through the origin, but the slope is less than unity, because the test image SNRs are proportionately smaller in SNR than the corresponding insert image SNRs. Fig. 11.5 (b) shows SNR of one of the insert images vs. corresponding averaged insert image SNRs. The slope of this line is close to unity. Note that one can get from (A) to (B) not by adding a constant value; rather one needs to multiply the slope by a constant value. [At the risk of stating the obvious, in this analogy, the insert images represent one treatment and the test image represents another treatment, and SNR is a unit normal distribution separation measure completely analogous to .]

# 11.13: Discussion/Summary

In the author's experience, the topic of sample-size estimation evokes some trepidation in non-statistician investigators engaging in ROC studies. Statisticians who understand the specialized techniques that have been developed for ROC studies may not be readily available, on when available, may lack sufficient clinical background to interact knowledgeably with the researcher. Lacking this resource the investigator looks to the literature for "similar studies" and follows precedents. It is not surprising that some published studies tend to cluster around similar numbers, e.g., about 5 readers and 50-100 cases. Sample-size methodology is a valuable tool since they potentially allow non-experts to plan reasonably powered ROC studies to answer questions like "is an image processing method better than another". However, proper usage of these tools requires understanding of how they work, or at the very least, to be able to cogently discuss the issues involved with expert statisticians.

Size estimation methodology for MRMC ROC studies has gone through several iterations and in the author's opinion. This started with work by Obuchowski4-6,21,22, which as noted earlier, see Chapter "ORH Analysis" §10.3.3, led to some consternation, because it predicted excessively large sample sizes, and also suggested that in addition to conducting a pilot study, one needed to estimate within-reader variability by repeating the interpretations with the same readers and same cases. As noted in 2011 by Hillis et al, the within reader component of variance does not need to be separately estimated7. Two updates by Hillis and colleagues followed, one3 in 2004 and the other in 20117. The latest paper, titled "Power Estimation for Multireader ROC Methods: An Updated and Unified Approach" still lacks fixed reader and fixed case analysis, and it has some errors, e.g., page 135, expression for . Given the number of parameters entering the computations, numerical errors are almost inevitable in a step-by-step description. This is the reason the author prefers the software approach, because software is consistent, if it is wrong it is consistently wrong; but it will consistently remember that  is supposed to be 7 and not 4, as in the cited example.

It is the author's preference, as in this chapter, not to mix the statistically well-known quantities "mean squares" with variance components in sample size formulae. Mean squares are not as "portable" as variance components: the latter are intrinsic quantities that can be exchanged between different researchers without having to know the details of how they ware calculated. With mean squares the connection depends on the sample size over which the mean squares were calculated, see Table 1 in Ref. 7. The intermixing of mean squares and variance components in publications, while of little consequence to statisticians, can make the formulae unnecessarily dense for the rest of us.

The main remaining issue is the selection of the effect-size, and since it appears as the square, it can lead to widely divergent sample-sizes. This is where a pilot study is helpful, but the results need to be interpreted with some caution. The fundamental problem is this: a clinically meaningful and technologically achievable effect-size, consistent with the pilot study, needs to be chosen. Good communication between the researcher and those familiar with the clinical picture is essential. Additionally, the author believes that the current practice of specifying effect-size in AUC units is not appropriate. A method of specifying effect-size as small, medium or large is proposed, which uses multiples of the separation parameter and makes clinical sense. This can be used in situations where the pilot study is believed to not provide good information, and rather than repeat the whole pilot study, one can with some justification propose an effect-size based on a multiple of the observed separation of the normal distributions describing the ROC. In addition, in the author's opinion, the chosen effect size should not greatly exceed that revealed by the pilot study.

For all its tortuous progress, current sample size methodology provides a principled approach to planning an ROC study. The approach, recommended in some circles, of using "magic numbers" like 6 readers and 50 cases, with no justification for their selection, other than the reputation of the person proposing it, is one the author cannot condone.

This concludes Part B of the book. It is time to move on to Part C, namely the FROC paradigm.

# 11.14: References

1.	ICRU. Statistical Analysis and Power Estimation. JOURNAL OF THE ICRU. Vol 82008:37-40.

2.	Cohen J. Statistical power analysis for the social sciences. 1988.

3.	Hillis SL, Berbaum KS. Power Estimation for the Dorfman-Berbaum-Metz Method. Acad Radiol. 2004;11(11):1260-1273.

4.	Obuchowski NA. Multireader, Multimodality Receiver Operating Characteristic Curve Studies: Hypothesis testing and Sample Size Estimation Using an Analysis of Variance Approach with dependent Observations. Acad Radiol. 1995;2:S22-S29.

5.	Obuchowski NA. Sample size calculations in studies of test accuracy. Statistical Methods in Medical Research. 1998;7(4):371-392.

6.	Obuchowski NA. Sample Size Tables For Receiver Operating Characteristic Studies. Am J Roentgenol. 2000;175(3):603-608.

7.	Hillis SL, Obuchowski NA, Berbaum KS. Power Estimation for Multireader ROC Methods: An Updated and Unified Approach. Academic Radiology. 2011;18(2):129-142.

8.	Van Dyke CW, White RD, Obuchowski NA, Geisinger MA, Lorig RJ, Meziane MA. Cine MRI in the diagnosis of thoracic aortic dissection. 79th RSNA Meetings. 1993.

9.	Franken EA, Jr., Berbaum KS, Marley SM, et al. Evaluation of a Digital Workstation for Interpreting Neonatal Examinations: A Receiver Operating Characteristic Study. Investigative Radiology. 1992;27(9):732-737.

10.	Kreaemer HC, Mintz J, Noda A, Tinklenberg J, Yesavage JA. Caution regarding the use of pilot studies to guide power calculations for study proposals. Arch Gen Psychiatry. 2006;63:484-489.

11.	Chakraborty DP. Prediction accuracy of a sample-size estimation method for ROC studies. Academic radiology. 2010;17:628–638.

12.	Roe CA, Metz CE. Dorfman-Berbaum-Metz Method for Statistical Analysis of Multireader, Multimodality Receiver Operating Characteristic Data: Validation with Computer Simulation. Acad Radiol. 1997;4:298-303.

13.	Beiden SV, Wagner RF, Campbell G. Components-of Variance Models and Multiple-Bootstrap Experiments: An Alternative Method for Random-Effects, Receiver Operating Characteristic Analysis. Academic Radiology. 2000;7(5):341-349.

14.	Chakraborty DP, Gupta KL, Barnes GT, Vitek JJ. Digital Subtraction Angiography Apparatus. Radiology. 1985;157:547.

15.	Chakraborty DP. Physical measures of image quality in mammography. Paper presented at: Proc. SPIE 2708, Medical Imaging 1996: Physics of Medical Imaging1996; Newport Beach CA.

16.	Chakraborty DP, Sivarudrappa M, Roehrig H. Computerized measurement of mammographic display image quality. Paper presented at: Proc SPIE Medical Imaging 1999: Physics of Medical Imaging1999; San Diego, CA.

17.	Chakraborty DP, Fatouros PP. Application of computer analyis of mammography phantom images (CAMPI) methodology to the comparison of two digital biopsy machines. Paper presented at: Proc SPIE Medical Imaging 1998: Physics of Medical Imaging; 24 July 1998, 1998.

18.	Chakraborty DP. Comparison of computer analysis of mammography phantom images (CAMPI) with perceived image quality of phantom targets in the ACR phantom. Paper presented at: Proc. SPIE Medical Imaging 1997: Image Perception; 26-27 February 1997, 1997; Newport Beach, CA.

19.	Chakraborty DP. Computer analysis of mammography phantom images (CAMPI). Proc SPIE Medical Imaging 1997: Physics of Medical Imaging. 1997;3032:292-299.

20.	Chakraborty DP. Computer analysis of mammography phantom images (CAMPI): An application to the measurement of microcalcification image quality of directly acquired digital images. Medical Physics. 1997;24(8):1269-1277.

21.	Obuchowski NA. Computing Sample Size for Receiver Operating Characteristic Studies. Inv Radiol. 1994;29(2):238-243.

22.	Obuchowski NA, McLish DK. Sample size determination for diagnostic accuracy studies involving binormal ROC curve indices. Statistics in Medicine. 1997;16:1529-1542.

Another name for vignette.