Chapter 14: Computation and meanings of empirical FROC FOM-statistics and AUC measures

# Table of contents

- Introduction

- Empirical AFROC FOM-statistic

- Empirical weighted-AFROC FOM-statistic

- Two Theorems

- Understanding the AFROC and wAFROC empirical plots

- Physical interpretation of AFROC-based FOMs

- Summary

- References

Online Supplementary Material

Online Appendix 14.A: Proof of Equivalence theorem for wAFROC

Online Appendix 14.B: Understanding the AFROC and wAFROC

Online Appendix 14.C: Summary of FROC FOMs

Online Appendix 14.D: Numerical demonstrations of FOM-statistic vs. AUC equivalences

# 14.1: Introduction

The previous chapter focused on empirical plots possible with FROC data. Examples are the FROC, AFROC, wAFROC, inferred ROC, etc. Expressions were given for computing operating points for each plot. Because of same conundrum faced with quoting sensitivity and specificity pairs, operating points are rarely used as figures of merit. Rather interest is in area measures derived from operating characteristics.

In this chapter, the empirical area under a plot is generically denoted , where the "oc" subscript denotes the applicable operating characteristic. For example, the area under the empirical weighted-AFROC is denoted . Calculating areas from operating points using planimetry or geometry is tedious. Needed are formulae for calculating them directly from ratings. An example of this was encountered before in Chapter 05, where it was shown that the area under the empirical ROC  plot equaled the Wilcoxon statistic.

A distinction is being made between empirical AUC under a plot, i.e., an area, and a FOM-statistic, generically denoted , that is computed directly from the ratings. The FOM-statistic is a scalar function of the ratings intended to quantify performance. While any function of the ratings is a possible FOM-statistic, whether a proposed function is useful depends upon whether it can be related to the area under a meaningful operating characteristic. This chapter derives formulae for FOM-statistics , which yield the same values as the empirical areas  under the corresponding empirical operating characteristics. The meanings of these FOM-statistics are discussed1.

Here is the organization of the chapter. Expressions for the empirical AFROC FOM-statistic  and the empirical weighted-AFROC FOM-statistic  are presented and their limiting values for chance-level and perfect performances are explored. Two important theorems are stated, whose proofs are in Online Appendix 14.A. The first theorem proves the equality between the empirical wAFROC FOM-statistic  and the area  under the empirical wAFROC plot. A similar equality applies to the empirical AFROC FOM-statistic  and the area  under the empirical AFROC plot. The second theorem derives an expression for the area under the straight-line extension of the wAFROC (or AFROC) from the observed end-point to (1,1), and explains why it is essential to include this area. A section explaining, with a small simulated-dataset, how NL and LL ratings and lesion weights determine the AFROC and wAFROC empirical plots follows. It shows explicitly that the wAFROC gives equal importance to all diseased cases, a desirable statistical characteristic, while the AFROC gives excessive importance to cases with more lesions. The small numbers of cases makes the explanations easier to follow and permits "hand-calculation" of the operating points. This is followed by physical interpretation of the corresponding AUCs / FOM-statistics. It shows explicitly how the comparisons implied in FOM-statistic properly credit and penalize the observer for correct and incorrect decisions, respectively. This section gives probabilistic meanings to the AFROC and wAFROC AUCs.

Detailed derivations of FOM-statistics applicable to the areas under the empirical FROC plot, the AFROC1 and wAFROC1 plots are not given. Instead, the results for all plots are summarized in Online Appendix 14.C, which shows that the definitions "work", i.e., the FOM-statistics yield the correct areas as determined by numerical integrations.

# 14.2: Empirical AFROC FOM-statistic

The empirical AFROC AUC was defined in §13.4.2 as the area under the empirical AFROC. The corresponding FOM-statistic  is defined2 in terms of a quasi-Wilcoxon statistic by (the kernel function  was defined in Eqn. (5.10)):

.

FROC notation is summarized in Table 13.1. The conditioning on z-samples greater than on equal to  ensures that only marked regions contribute to . In Chapter 13, the relevant formulae ensured that only marked regions contributed to operating points. A similar requirement needs to be imposed in expressions for FOM-statistics.

The FOM-statistic  achieves its highest value, unity, if and only if every lesion is rated higher than every mark on every non-diseased case, for then the  function always yields unity, and the summations yield:

.

If, on the other hand, every lesion is rated lower than every mark on every non-diseased case, the  function yields zero, and the FOM-statistic is zero. Therefore,

.

The range of , Eqn. , is twice that of , i.e., 0.5 to 1. This has the important consequence that treatment or modality related differences between  (i.e., effect sizes) tend to be larger relative to the corresponding ROC effect sizes. This is explained further in Chapter 18. Eqn.  is another reason why the "chance diagonal" of the AFROC corresponding to AUC = 0.5, does not reflect chance-level performance. An area under the AFROC equal to 0.5 is actually reasonable performance, being smack in the middle of the allowed range. An example of this was given in §13.4.2.2 for the case of an expert radiologist who does not mark any cases.

# 14.3: Empirical weighted-AFROC FOM-statistic

The empirical weighted-AFROC plot and lesion weights were defined in §13.7. The empirical weighted-AFROC FOM-statistic3 is defined in terms of a quasi-Wilcoxon statistic:

.

The weights obey the constraint:

.

If not explicitly qualified as a weighted-AFROC FOM-statistic, it is understood that one means the conventional (not-weighted) AFROC FOM-statistic defined by Eqn. . In the special case of one lesion per diseased case, the two definitions are identical. However, for multiple lesions per case, they are not, as one cannot set the weights equal to unity, e.g., for equally weighted lesions,

.

For equally weighted lesions, for a case with three lesions, each weight equals one-third (1/3).

# 14.4: Two Theorems

The area  under the wAFROC plot is obtained by summing the areas of individual trapezoids defined by drawing vertical lines from each pair of adjacent operating points to the x-axis. A sample plot is shown Fig. 14.1.

Fig. 14.1: An example wAFROC plot; from left to right, the two shaded areas correspond to  and , respectively, defined in the text below. This plot was produced by mainProofPlot.R.

The operating point labeled  has coordinates  given by Eqn. (13.11) and Eqn. (13.17), respectively, reproduced here for convenience:

.

.

As usual, the summation in Eqn.  is conditioned on marked NLs. The conditioning is unnecessary for LLs. Online Appendix 14.A proves the following theorems:

## 14.4.1: Theorem 1

The area  under the empirical wAFROC plot, where  are defined by Eqn.  and Eqn. , equals the weighted-AFROC FOM-statistic  defined by Eqn. :

.

This is the FROC counterpart of Bamber's Wilcoxon – empirical ROC AUC equivalence theorem4, derived in Chapter 05.

## 14.4.2: Theorem 2

The areaunder the straight-line extension of the wAFROC from the observed end-point  to (1,1) is given by:

.

According to Eqn. , increases as  decreases, i.e., as more non-diseased cases are not marked and as  increases, i.e., as more lesions, especially those with greater weights, are marked. Both observations are in keeping with the behavior of a valid FOM. Failing to include the area under the straight-line extension results in not counting the positive contribution to the FOM of unmarked cases, each of which represents a perfect decision. For a perfect observer whose operating characteristic is the vertical line from (0,0) to (0,1), the area under the straight-line extension comprises the entire AUC for the wAFROC. Excluding it would predict zero AUC for a perfect observer, which is illogical.

# 14.5: Understanding the AFROC and wAFROC empirical plots

The code for this example is in file mainExamples.R in Online Appendix 14.B. Parameters of the simulation are , , ,  and . It simulates a dataset consisting of four non-diseased cases and four diseased cases. The first two diseased cases have one lesion each, and the remaining two have two lesions each. Clicking source yields §14.5.1 and two plots, an AFROC and a wAFROC, similar to Fig. 14.3 and Fig. 14.5, but without the labeling.

### 14.5.1: Code output

> source('~/book2/04 C FROC analysis/C3 Meanings of FROC FOMs/software/mainExamples.R')

AFROC AUC =  0.7708333

wAFROC AUC =  0.7875

Use the commands in §14.5.2 to get the number of lesions per case and the ratings in the NL and LL arrays.

### 14.5.2: Code snippets

Browse[2]> Lk2

[1] 1 1 2 2

Browse[2]> frocData$NL[1,1,,]

[,1]      [,2]

[1,]       -Inf      -Inf

[2,]  0.4874291      -Inf

[3,]  0.7383247 0.5757814

[4,] -0.3053884      -Inf

[5,]  1.5117812      -Inf

[6,]       -Inf      -Inf

[7,]       -Inf      -Inf

[8,]       -Inf      -Inf

Browse[2]> frocData$LL[1,1,,]

[,1]    [,2]

[1,]  0.8523430    -Inf

[2,] -0.2146999    -Inf

[3,]  1.5884892    -Inf

[4,]  2.9438362 1.98381

The length of the third dimension of the NL array is eight (4 non-diseased + 4 diseased cases). The fifth sequential case corresponds to NLs on the first diseased case, etc. The simulated z-samples displayed in §14.5.2 are shown in Table 14.1 for non-diseased and in Table 14.2 for diseased cases. The columns labeled  list the case-location indexing subscripts, the columns labeled  list the corresponding z-samples, when realized and otherwise NAs are listed. Column 5 in Table 14.1 illustrates the conversion of the NL z-samples to FP z-samples according to the highest-rating assumption (the first non-diseased case illustrates the rule that in the absence of any marks the FP rating is -∞). The tables show that the simulator did not realize any z-sample on the first non-diseased case (alternatively, if it did, the z-sample(s) fell below ; one cannot tell the difference), and for the second lesion on the third diseased case. Because non-diseased cases have no lesions, all z-samples listed in Table 14.1 are NLs. In contrast, in Table 14.2, each case can generate NLs and LLs. The second column of Table 14.2 lists the number of lesions per diseased case . Columns 3 and 4 illustrate NL indexing and z-samples and columns 5 and 6 illustrate LL indexing and z-samples. Column 8 illustrates the conversion of the NL and LL z-samples to TP z-samples according to the highest-rating assumption. The last columns of Table 14.1 and Table 14.2 label the correspondences of the z-samples to the operating points shown in Fig. 14.3 and Fig. 14.5. Where applicable, unrealized z-samples are indicated by asterisks.

Table 14.1: Simulated data used in example plots for non-diseased cases yielding the green FP circles in Fig. 14.2 and Fig. 14.4; * = unmarked; NA = not applicable z-sample.

| Non-diseased Case | NLs |  |  | FPs | Operating points in Fig. 14.3 and Fig. 14.5 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1111 | NA | 11 | -∞ | * |
|  | 1121 | NA |  |  |  |
| 2 | 2111 | 0.487 | 21 | 0.487 | F |
|  | 2121 | NA |  |  |  |
| 3 | 3111 | 0.738 | 31 | 0.738 | E |
|  | 3121 | 0.576 |  |  |  |
| 4 | 4111 | -0.305 | 41 | -0.305 | H |
|  | 4121 | NA |  |  |  |

Table 14.2: Simulated data used in example AFROC and wAFROC plots for diseased cases yielding the red LL circles in Fig. 14.2 and Fig. 14.4; NA = not applicable; * = unmarked lesion; note that the z-samples listed under NLs are ignored in AFROC and wAFROC analyses, but are used in (inferred) ROC analysis.

| Diseased Case |  | NLs |  | LLs |  |  | TPs | Lesion weights | Operating points in Fig. 14.3 and Fig. 14.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1211 | 1.512 | 1212 | 0.852 | 12 | 1.512 | 1 | D |
|  |  | 1221 | NA | 1222 | NA |  |  | NA |  |
| 2 | 1 | 2211 | NA | 2212 | -0.215 | 22 | -0.215 | 1 | G |
|  |  | 2221 | NA | 2222 | NA |  |  | NA |  |
| 3 | 2 | 3211 | NA | 3212 | 1.588 | 32 | 1.588 | 0.6 | C |
|  |  | 3221 | NA | 3222 | NA |  |  | 0.4 | * |
| 4 | 2 | 4211 | NA | 4212 | 2.944 | 42 | 2.944 | 0.4 | A |
|  |  | 4221 | NA | 4222 | 1.984 |  |  | 0.6 | B |

### 14.5.3: The AFROC plot

In Fig. 14.2, FPs and LLs, represented by green and red circles, respectively, are shown ordered, from left to right, with higher z-samples to the right (this is subsequently referred to as a "one-dimensional" depiction of the data). Each circle is labeled using the  notation. For example, the right-most red circle corresponds to the LL z-sample originating from the first lesion in the fourth diseased case, i.e., . Consistent with Table 14.1, there are three green circles (FPs) not counting , which occurs at z = -∞ representing the first non-diseased case with no marks. Likewise, consistent with Table 14.2, there are five red circles (LLs) not counting = -∞ representing the unmarked second lesion on the third diseased case.

Fig. 14.2: A one-dimensional depiction of the data in Table 14.1 and Table 14.2, showing z-samples used for plotting the AFROC; the red circles correspond to latent lesion localizations (LLs) and the green to latent false positives (FPs). This plot was generated by main1DPlotAfroc.R.

Starting from +∞, moving a virtual threshold continuously to the left generates the AFROC plot, Fig. 14.2. As each FP is crossed, the operating point moves to the right by

.

As each LL is crossed, the operating point moves up by

.

Since it has one lesion, crossing the z-sample for the first case would result in an upward movement of 1/6, and likewise for the second case. Since the third case contains two lesions, crossing the corresponding z-samples would result in a net upward movement of the operating point by 1/3. This behavior shows explicitly that the non-weighted method gives greater importance to diseased cases with more lesions, i.e., such case make a greater contribution to AUC. The jumps from lesions in the same case need not be contiguous; they could be distributed, with intervening jumps from lesions on other cases but eventually the jumps will occur and contribute to the net upward movement. As an example, the jumps due to the two lesions on the fourth case are contiguous: see points A and B, in Fig. 14.3. However, the jumps due to the two lesions on the 3rd case are not contiguous: the first lesion gives the point C, but the unmarked lesion on this case, indicated by the asterisk in Table 14.2, eventually contributes when the operating point moves diagonally from point H to (1,1).

Fig. 14.3: The empirical AFROC plot for the data shown in Table 14.1 and Table 14.2. The labels correspond to the last columns of the tables. The corresponding one-dimensional depiction is Fig. 14.2. The area under the empirical plot is 0.7708. This plot was generated by mainPlotAfroc.R.

### 14.5.4: The weighted-AFROC (wAFROC) plot

Fig. 14.4, which is analogous to Fig. 14.2, is a one-dimensional depiction of the data in Table 14.1 and Table 14.2 but this time the lesion weights, shown in Table 14.2, are incorporated, as indicated by varying the size of each red circle to indicate its weight (in Fig. 14.2 all red circles were of the same size). In addition, each lesion is labeled with its rating and weight. The listed weights in Table 14.2 were assigned arbitrarily subject to the rule that the weights on a case must sum to unity.

Fig. 14.4: A one-dimensional depiction of the data in Table 14.1 and Table 14.2, including the weights, showing z-samples used for plotting the weighted-AFROC plot; the sizes of the red circles code the lesions weights (the lesion-specific weights are shown below each z-sample). This plot was generated by main1DPlotwAfroc.R.

Moving a virtual threshold continuously to the left generates the wAFROC plot, Fig. 14.5. The movement of the operating point in response to crossing FPs is the same as before. However, as each LL is crossed the operating point moves up by an amount that depends on the lesion:

.

Since the first two diseased cases have one lesion each (i.e., unit weights), crossing the corresponding z-samples results in upward jumps of 0.25, Fig. 14.5 (C to D and F to G). According to the weights in Fig. 14.4, crossing the z-sample of the first lesion in the third diseased case, results in an upward jump of 0.6/4. That from the second lesion in the same case results in an upward jump of 0.4/4, for a net upward jump of the third case of 0.25, the same as for the first two diseased cases. Likewise crossing the z-samples of the two lesions in the 4th disease case results in upward jump of 0.4/4 = 0.1 (O to A), for the 1st lesion and 0.6/4 = 0.15 (B to C), for the 2nd lesion, for a net upward jump of ¼, which is the same as for the first three diseased cases. This shows explicitly that the weighting method gives each diseased case the same importance, regardless of the number of lesions in it, a property not shared by the area under the AFROC.

Fig. 14.5: The empirical weighted-AFROC (wAFROC) plot for the data shown in Table 14.1 and Table 14.2. The operating point labels correspond to the last columns of the tables. The corresponding one-dimensional plot is Fig. 14.4. The area under the wAFROC is 0.7875. This plot was generated by mainPlotwAfroc.R.

# 14.6: Physical interpretation of AFROC-based FOMs

From the preceding sections, it is seen that the AFROC-based trapezoidal plots consist of upward and rightward jumps, starting from the origin (0,0) and ending at (1,1). This is true regardless of whether the z-samples are binned or not: i.e., at the "microscopic" level the jumps always exist. Each upward jump is associated with a LL rating exceeding a virtual threshold. Each rightward jump is associated with a FP rating exceeding the threshold. Upward jumps tend to increase the area under the AFROC-based plots and rightward jumps tend to decrease it. This makes physical sense in terms of correct decisions being rewarded and incorrect ones being penalized, and can be seen from two extreme-case examples. If there are only upward jumps, then the trapezoidal plot rises from the origin to (0,1), where all lesions are correctly localized without any generating FPs and performance is perfect – the straight-line extension to (1,1) ensures that the net area is unity. If there are only horizontal jumps, that takes the operating point from the origin to (1,0), where none of the lesions are localized and every non-diseased image has at least one NL mark, representing worst possible performance. Here, despite the straight line extension to (1,1) the net area is zero.

## 14.6.1: Physical interpretation of area under AFROC

The area under the AFROC has the following physical interpretation: it is the fraction of LL vs. FP z-sample comparisons where the LL sample is equal (counting as half a comparison) or greater (counting as a full comparison) than the FP z-sample. From Table 14.1 and Table 14.2, there are four FPs and six LLs for 24 possible comparisons. Inspection of the tables reveals that there are 4 x 4 = 16 comparisons contributing ones, two comparisons (from the 2nd diseased case) contributing ones, and one comparison (from the 2nd lesion on the 3rd diseased case) contributing 0.5, which sum to 18.5. Dividing by 24 yields 18.5/24 = 0.7708, the empirical AFROC-AUC, §14.5.1. In probabilistic terms:

The area under the AFROC is the probability that a lesion is rated higher than any mark on a non-diseased case.

## 14.6.2: Physical interpretation of area under wAFROC

The area under the wAFROC has the following physical interpretation: it is the lesion-weight adjusted fraction of diseased cases vs. non-diseased case comparisons where LL z-samples are equal (counting as half a comparison times the weight of the lesion in question) or greater (counting as a full comparison times the weight of the lesion) than FP z-samples. Note that there are still 24 LL vs. FP comparisons but the counting proceeds differently. The fourth diseased case contributes 0.4 x 4 + 0.6 x 4, i.e., 4 (compared to 8 in the preceding example). The third diseased case contributes 0.6 x 4 + 0.4 x 0.5, i.e., 2.6 (compared to 4.5 in the preceding example). The second diseased case contributes 1 x 2 = 2 (compared to 2 in the preceding example), and the first diseased case contributes 1 x 4 = 4 (compared to 4 in the preceding example). Summing these values and dividing by 16 (the total number of diseased cases vs. non-diseased cases comparisons) one gets 12.6/16 = 0.7875, which is the area under the wAFROC, §14.5.1. In probabilistic terms:

The area under the weighted-AFROC is the lesion-weight adjusted probability that a lesion is rated higher than any mark on a non-diseased case.

# 14.7: Discussion / Summary

The primary aim of this chapter was to develop expressions for FOMs (i.e., functions of ratings) and show their equivalences to the empirical AUCs under corresponding operating characteristics. Unlike the ROC, the AFROC and wAFROC figures of merit are represented by quasi-Wilcoxon like constructs, not the well-known Wilcoxon statistic5.

The author is aware from users of his software that their manuscript submissions have sometimes been held up with the critique that the meaning of the AFROC FOM-statistic is "not intuitively clear"6. An example was given in a previous chapter. This is one reason the author has tried to make the meaning clear, perhaps at the risk of making it painfully clear. Clinical interpretations do not always fit into convenient easy to analyze paradigms. Not understanding something is not a reason for preferring a simpler method. Use of the simpler ROC paradigm to analyze location specific tasks results in loss of statistical power and sacrifices better understanding of what is limiting performance. It is unethical to analyze a study with a method with lower statistical power when one with greater power is available7-9. The title of the paper by Halpern et al is "The continuing unethical conduct of underpowered clinical trials". The AFROC FOM-statistic was proposed in 1989 and it has been used, at the time of writing, in over 107 publications.

The subject material of this chapter is not that difficult. However, it does require the researcher to be receptive an unbiased. Dirac addressed an analogous then-existing concern about quantum mechanics, namely it did not provide a satisfying "picture" of what is going on, as did classical mechanics. To paraphrase him, the purpose of science (quantum physics in his case) is not to provide satisfying pictures but to explain data. FROC data is inherently more complex than the ROC paradigm and one should not expect a simple FOM-statistic. The detailed explanations given in this chapter should allow one to understand the wAFROC and AFROC FOMs.

A misconception regarding the wAFROC FOM-statistic is that the weighting may sacrifice statistical power and render the method equivalent to ROC analysis in terms of statistical power. Analysis of clinical datasets and simulation studies suggests that this is not the case; loss of power is minimal. As noted earlier, the highest rating carries more information than a randomly selected rating.

Bamber's equivalence theorem led to much progress in non-parametric analysis of ROC data. The proofs of the equivalences between the areas under the AFROC and wAFROC and the corresponding quasi-Wilcoxon statistics provide a starting point. To realize the full potential of these proofs, similar work like that conducted by DeLong et al10 is needed for the FROC paradigm. This work is not going to be easy; one reason being the relative dearth of researchers working in this area, but it is possible. Indeed work has been published by Popescu11 on non-parametric analysis of the exponentially transformed FROC  (EFROC) plot which, like the AFROC and wAFROC, is completely contained within the unit square. This work should be extended to the wAFROC. For reasons stated in Chapter 13, non-parametric analysis of FROC curves12-14 is not expected to be fruitful.

Current terminology prefixes each of the AFROC-based FOMs with the letter "J" for Jackknife. The author recommends dropping this prefix, which has to do with significance testing procedure rather than the actual definition of the FOM-statistic. For example, the correct way is to refer to the AFROC figure of merit, not the JAFROC figure of merit. For continuity, the software packages implementing the methods are still referred to as JAFROC (Windows) or RJAfroc (cross-platform, open-source).

To gain deeper insight into the FROC paradigm, it is necessary to look at methods used to measure visual search, the subject of the next chapter.

# 14.8 References

1.	Chakraborty DP, Zhai X. On the meaning of the weighted alternative free-response operating characteristic figure of merit. Medical physics. 2016;43(5):2548-2557.

2.	Chakraborty DP, Yoon HJ. JAFROC analysis revisited: figure-of-merit considerations for human observer studies. Proc SPIE Medical Imaging: Image Perception, Observer Performance, and Technology Assessment. 2009;7263:72630T.

3.	Chakraborty DP, Berbaum KS. Observer studies involving detection and localization: Modeling, analysis and validation. Med Phys. 2004;31(8):2313-2330.

4.	Bamber D. The area above the ordinal dominance graph and the area below the receiver operating characteristic graph. Journal of Mathematical Psychology. 1975;12(4):387-415.

5.	Wilcoxon F. Individual Comparison by Ranking Methods. Biometrics. 1945;1:80-83.

6.	Hillis SL, Chakraborty DP, Orton CG. ROC or FROC? It depends on the research question. Medical Physics. 2017.

7.	Lerman J. Study design in clinical research: sample size estimation and power analysis. Canadian journal of anaesthesia. 1996;43(2):184-191.

8.	Halpern SD, Karlawish JH, Berlin JA. The continuing unethical conduct of underpowered clinical trials. Jama. 2002;288(3):358-362.

9.	Breau RH, Carnat TA, Gaboury I. Inadequate statistical power of negative clinical trials in urological literature. The Journal of urology. 2006;176(1):263-266.

10.	DeLong ER, DeLong DM, Clarke-Pearson DL. Comparing the Areas Under Two or More Correlated Receiver Operating Characteristic Curves: A Nonparametric Approach. Biometrics. 1988;44:837-845.

11.	Popescu LM. Nonparametric signal detectability evaluation using an exponential transformation of the FROC curve. Medical physics. 2011;38(10):5690-5702.

12.	Bandos AI, Rockette HE, Song T, Gur D. Area under the Free-Response ROC Curve (FROC) and a Related Summary Index. Biometrics. 2008;65(1):247-256.

13.	Samuelson FW, Petrick N, Paquerault S. Advantages and examples of resampling for CAD evaluation. Paper presented at: Biomedical Imaging: From Nano to Macro, 2007. ISBI 2007. 4th IEEE International Symposium on Biomedical Imaging2007; Arlington VA, USA.

14.	Samuelson FW, Petrick N. Comparing image detection algorithms using resampling. IEEE International Symposium on Biomedical Imaging: From Nano to Micro. 2006:1312-1315.

Quasi: seemingly; apparently, but not really

Here is another example of how the maximum value carries more information than a randomly selected value. If the maximum rating over all NLs on a case is smaller than a specified value, then every NL on the case must be rated smaller than the specified value.

https://www.researchgate.net/publication/311102039_Publications_that_have_used_JAFROC_software_as_of_112916

"In answer to the first criticism it may be remarked that the main object of physical science is not the provision of pictures, but is the formulation of laws governing phenomena and the application of these laws to the discovery of new phenomena. If a picture exists, so much the better; but whether a picture exists or not is a matter of only secondary importance." P.A.M. Dirac, Principles of Quantum Mechanics, 4th edition, Chapter 1. http://www.informationphilosopher.com/solutions/scientists/dirac/chapter_1.html