# Online Appendix 18.A: Instructive correspondence with a user (identifiers suppressed)

Dear Dev,

We are conducting a reader study on xx where readers are freely asked to mark and score xx lesions. We are using your JAFROC software (v4.2.1) to analyze the data, but we are encountering some doubts that we hope you can kindly help us with.

Attached you can find the original data, the .xls sheet, as well as the results of using JAFROC-2, and JAFROC-2-WEIGHTED (following the definition from your 2004 Med Phys paper). We have xx cases (xx abnormal), xx modalities and xx readers. We treat everything as random.

As you can see, WEIGHTED analysis results into non-significant differences between modalities, as opposed to the non-weighted analysis. Is there any particular reason for that?  Only three abnormal cases have 2 lesions, for the rest is 1. On your readme file you write this:

Weighted vs. non-weighted: Weighting assures that all abnormal cases get equal importance, regardless of the number of lesions on them, a desirable statistical characteristic. The author has encountered a situation in nuclear medicine bone scans where the number of lesions can vary over a very large range (1 – 100) and in this situation only weighted analysis should be reported. Based on our experience, there is little difference between the two analyses when the number of lesions varies from 1-3. There is some loss of statistical power in using weighted over non-weighted figures-of-merit, but the benefits of localization (vs. ROC analysis) are largely retained. Unless there are clinical reasons for doing otherwise, equal weighting is recommended (assigning zero weights to all images achieves this).

A second doubt is that, when I use 3 modalities (instead of 4) and 6 readers (5 from above, plus one who only read 3/4 modalities), get significant differences. I cannot really understand why, so if you have some quick thoughts we would gladly hear them.

Finally, JAFROC calculates a figure-of-merit defined as the probability that a lesion is rated higher than any mark on a normal image. It is estimated using the non-parametric Mann-Whitney-Wilcoxon U-statistic applied to the lesion ratings and the highest rating on normal images, where unmarked lesions and unmarked normal images are assigned the negative infinity rating. However, on the result files that I attach, which are generated by the software, these FOMs are still labeled as "Area". Is this corrected, am I missing something or it is a small mistake of the software?

Thank you very much for your time!

Dear xx,

I have examined your dataset and there is nothing unusual about your results. The p-value is a realization of a random variable and it is extremely sensitive to small differences in the observed F-statistic; one expects wAFROC to give slightly greater p-value, which you observe, but the values of wAFROC and AFROC are actually quite close; nothing to get excited about.

If you apply two sequential tests to get the result that you prefer, you should, in all honesty, be using an alpha of 0.025 (Bonferroni correction), according to which neither method rejects the NH. Or you could simply not report that you tried wAFROC first, which I would not advise, especially when you are beginning your research career.

There is nothing wrong with reporting a negative result, as long as you quote a CI for the differences and variance components, output by the software. There is a paper by Metz on the importance of confidence intervals when failing to reject the NH. These results in the archival literature are helpful to others, especially for sample size estimation for future studies.

Results for all three methods as excel files are attached.

There is no mistake in the software; the “area” that you refer to is standard format used in all ROC software, including U of Iowa and U of Chicago software, it is generic for the area measure; when organizing a tabular output it is helpful to have short names. A line in the output file gives the precise definitions:

Analysis method: WEIGHTED JAFROC FOM DBM-MRMC SIGNIFICANCE TESTING

Analysis method: JAFROC FOM DBM-MRMC SIGNIFICANCE TESTING

Dev

# Online Appendix 18.B: FROC sample size estimation using ORH analysis

Code mainwAFROCPowerORH.R is example using the ORH method to get the needed FOM variance components. Source it to get exactly the same output as in Table 18.4 and Fig. 18.8.

In the first call to SsPowerGivenJK, line 133, method is specified as "ORH" and it is followed by 5 arguments and K*, the number of cases in the pilot study. The supplied variance components are, in order, , ,,  and. [They were calculated by a previous call to StSignificanceTesting() , line 111, with method specified as "ORH" and FOM set to "HrAuc".] Also, effectSize is specified as effectSizeROC[i]. The second call, line 152, looks very similar but note the usage of variance components calculated by a previous call to StSignificanceTesting() [line 116, with FOM set to "wAFROC"]. Also effectSize is specified as effectSizewAFROC[i].

The importance of consistency in units cannot be overemphasized.

For an ES-ROC of 0.03, the power of the ROC method is about 18.5% while that of the wAFROC method is about 60.9%, the same as obtained by DBMH analysis. The reader should experiment with other datasets to become familiar with the method.

Finally, mainPowerObservedEffectSize.R calculates the expected power for the observed ROC and wAFROC effect-sizes. Source it to get the following output.

> source('~/Dropbox/RJafrocDownloads/book/Ch18/software/mainPowerObservedEffectSize.R')

NH i1 =  1 NH i2 =  2

JTest =  5 KTest =  200

ROC effect-size = , -0.0051 wAFROC effect-size = , -0.00685625 Power ROC, wAFROC: 0.05669581 , 0.0649482

It is not surprise that the differences between treatments 1 and 2 were not significant by either ROC or wAFROC analyses. The observed effect-sizes (0.0051 and 0.0069) are simply too small (the sign is not relevant). [The observed effect-sizes are realization of random variables – therefore, one should not expect the observed wAFROC effect-size to be exactly 2.18 times the observed ROC effect-size.]

With the trapezoidal AUC, the two methods will yield exactly the same results.