Chapter 21: The bivariate binormal model

# 21.1: Introduction

Until now, the focus for the ROC paradigm has been on the single rating per case scenario. This applies when a single reader interprets a set of cases in a single modality using the ROC paradigm. Of greater clinical interest is a study where a group of readers interprets a common set of cases in multiple modalities, termed the MRMC scenario. This is because such a study, properly analyzed, yields results that are expected to generalize to the populations of readers and cases from which the study readers and cases were sampled. However, before one can understand several readers interpreting a common case set in multiple modalities, a good place to start is two readers interpreting a common set of cases. Historically, the field progressed from this starting point (the DBMH and ORH analyses methods came after the method to be described here was developed, ca. 1980s). One might wonder why even bother describing the older approach, since methods for analyzing MRMC datasets have already been described. The answer is that before one can truly understand multiple-treatment multiple-reader studies, which permit a large number of pairings, without at least learning the history of analysis of data consisting of a single pairing.

From the measurement point of view the analyst is dealing with two ratings per case, one per reader, a key difference from the ROC paradigm we have been considering so far, which yielded one rating per case. An analogous situation applies when one has a single reader interpreting cases in two modalities. From the measurement point of view the analyst is again faced with two ratings per case, one per modality.

In Chapter 06 the univariate binormal model was described, which is appropriate for modeling the single rating per case situation (the "binormal" comes from the two truth states, not two modalities). From the previous discussion, it is evident that this needs to be extended to a bivariate model, which would describe studies where each case yields two ratings. Because one has dual interpretations of the same case, the two ratings are correlated. On especially easy non-diseased cases, the two ratings will both tend to be low (e.g., 1’s) and on especially easy diseased cases, the two ratings will both tend to be high (e.g., 5’s) and on difficult cases, both ratings are likely to be in the ambiguous range (e.g., 3’s). This type of tandem behavior is unlikely to be observed if the two interpretations did not come from the same case.

Here is the plan for this chapter. First, the bivariate extension of the univariate binormal model is described. Since most of the readers of this book are expected to not experts in statistics, bivariate sampling will be described at a relatively simple level. Visualization of the resulting probability density functions (pdfs) is demonstrated, which allows the reader to interactively view them, which should allow a better understand of this relatively more complex distribution. Estimation of parameters of the bivariate model from ratings data is addressed. The usage of free software implementing this procedure, i.e., CORROC2, is then described. The data format used by CORROC2 is described in some detail as well as how to interpret the output of the program. Since this software runs on Windows, material is provided to allow the user to run it on a MAC (most of this necessary digression, since the author is using OS X, is in an Online Appendix). This digression is not needed if one is using a Windows platform. Finally, the application of the software to clinical datasets is described. A recent advance that solves degeneracy problems associated with CORROC2 is described, namely CORCBM, for correlated CBM. We conclude with a discussion of the applications of CORROC2.

# 21.2: The Bivariate binormal model

In a single-modality single-reader ROC study the modality and reader indices are superfluous. Each z-sample can be thought of as arising from a binormal model with mean parameter, where t the truth index (t = 1, 2). The Z-samples of diseased cases are distributed . Non-diseased case Z-samples are distributed . In addition, for a ratings study, one needs thresholds . Recall, from Chapter 04, that for an R rating task with allowed ratings r = 1, 2, ..., R, one needs R-1 thresholds . Defining  and  as negative and positive infinity, respectively, the decision rule is to label a case with rating r if the realized z-sample satisfies.

In the two-modality scenario, each case yields two Z-samples, , where i = 1, 2 represent the two modalities, or generically, two "conditions". Additionally, the threshold vectors have condition dependencies,: i.e., there exist two sets of thresholds  and , assuming R ratings in one condition and S ratings in the other. The pairs  and  are defined as negative and positive infinities, respectively. The rating depends on the condition. The decision rule is to label a case in condition 1 with rating  if , and the same case in condition 2 is labeled with rating  if .

In the bivariate binormal model the decision variable is , where the i-subscript corresponds to the two modalities and the t subscript corresponds to the two truth states. The correlated ratings pairs and , corresponding to Z-samples from non-diseased (t = 1) and diseased cases (t = 2), respectively, are abbreviated, using vector notation, to :

.

According to the bivariate binormal model,  is sampled from , the bivariate normal distribution with mean and covariance matrix :

.

In Eqn. , the symbol  denotes a (column) vector  of length two from the bivariate normal distribution with mean and covariance matrix . Each case yields two samples, i.e., the vector has two rows, corresponding to the two modalities. The subscript 2 on distinguishes the bivariate distribution from the corresponding univariate distribution N (which has an implicit 1 subscript). The vectors and in Eqn.  are defined by:

.

The indices are it, i.e., modality followed by the truth index. The equations state that the two non-diseased distributions, one per modality, are both centered on zero, while the diseased distribution for modality 1 is centered at , shorthand for , while that for modality 2 is centered at , which is shorthand for .

The covariance matrices  and  in Eqn.  are defined by (also called variance-covariance matrices, but the author will stick with the shorter term):

.

The quantities in Eqn.  are defined as follows. For non-diseased cases, the Pearson correlation coefficient of the pairs of Z-samples is  and the corresponding value for diseased cases is. The ones along the diagonal of  confirm that the variances of the two samples for non-diseased cases are individually equal to unity. The corresponding variances for diseased cases are and .

The sampling model depicted in Eqn.  is a natural extension to two dimensions of the univariate binormal model described in Chapter 06, namely:

,

where,

.

The univariate model yields one Z-sample per case: samples for non-diseased cases have mean zero and unit variance; samples for diseased cases have mean and variance. In contrast the bivariate model yields two Z-samples per case, where for either modality, the samples for non-diseased cases have zero mean and unit variance, and the correlation coefficient of the two samples per non-diseased case is. For diseased cases, the samples for the first modality have mean  and variance , while for the second modality, the samples have mean  and variance  and the correlation coefficient of the two samples per diseased case is. To summarize, the parameters describing the bivariate model, not including the modality dependent thresholds needed for binning the data, are. The total number of parameters is 6: two means for the diseased cases, two variances for the diseased cases and two correlation coefficients.

In Online Appendix 21.A.1, file mainBivariateSampling.R, illustrates the bivariate sampling model. Specific values, selected arbitrarily, are assigned to the parameters as per Eqn. . The function mvrnorm() (to be understood as  “multivariate version of rnorm()") is used to generate the bivariate samples. The code generates N = 10,000 samples the bivariate distribution and the corresponding means and standard deviations were calculated.

.

Sourcing the file yields the following output (21.2.1) and Figure 21.1a:

### 21.2.1: Code Output

> source(...)

expected means of non-diseased cases =  0 0

expected correlation of non-diseased cases =  0.3

expected covariance of non-diseased cases =

[,1] [,2]

[1,]  1.0  0.3

[2,]  0.3  1.0

observed means of non-diseased cases =  -0.002791453 -0.007749206

observed correlation of non-diseased cases =  0.3194812

observed covariance of non-diseased cases =

[,1]      [,2]

[1,] 1.0051009 0.3225885

[2,] 0.3225885 1.0143726

expected means of non-diseased cases =  1.5 2

expected correlation of non-diseased cases =  0.6

expected covariance of non-diseased cases =

[,1] [,2]

[1,]  1.0 0.90

[2,]  0.9 2.25

observed means of diseased cases =  1.509351 2.009172

observed correlation of diseased cases =  0.6100278

observed covariance of diseased cases =

[,1]      [,2]

[1,] 1.0247098 0.9295477

[2,] 0.9295477 2.2659128

The output lists the expected values (i.e., based on the known parameter values) for non-diseased cases, followed by the observed values, and similarly for the diseased cases. Agreement between expected and observed values is good – this is due to the very large number of samples (10,000) over which the estimates are calculated. This exercise should give one some understanding of the bivariate sampling, the parameters needed to describe the samples and estimation of the parameters of the bivariate model from the observed samples. Section §21.4 below, on visualizing the bivariate pdf, will reinforce the reader's understanding.

# 21.3: The multivariate density function

The multivariate (specifically p-variate) probability density function (pdf) in p-dimensions is defined by

.

Here is a length-p column vector at which one wishes to evaluate the pdf; its mean is  (also a length-p column vector), the p x p covariance matrix is ,  is the determinant of the covariance matrix and T is the transpose operator. The reader should be convinced that the numerator of the exponent is a scalar. Eqn.  breaks down if the determinant of the covariance matrix (appearing in the denominator of the factor outside the exponent) is zero, consistent with the fact that the inverse matrix, appearing in the numerator of the exponent, only exists if the determinant is non-zero.

For the special case of a bivariate normal distribution this is equivalent to1:

.

Where

.

In the univariate case (i.e., p = 1),  in Eqn.  is replaced by , and the above expression reduces, as expected, to the well-known pdf of the normal distribution :

.

With some difficulty, if one is new to R, one could code Eqn.  (along the way, inverting the matrix will be but one minor challenge), but as one might expect, someone has already done it. The function is called dmvnorm(), to be understood as “density of multivariate normal distribution”, and it is available via a package mvtnorm(). Online Appendix 21.B.2 illustrates computation of the pdf of the multivariate normal distribution at a specified point. The reader should view and source file MainBivariateDensity.R.

## 21.3.1: Code output

> source(...)

rho = 0.5 , var1 = 1.5 , var2 = 1.5

density of bivariate normal at 0.1, 0.2 =  0.1208948

# 21.4: Visualizing the bivariate density function

In understanding the bivariate distribution, it is helpful to visualize the pdfs. Since one is dealing with a vector  of length 2, one needs two axes to select  and a third axis to depict the pdf at the selected , i.e., one needs a 3D plot on 2D paper. Various visualization techniques have been developed and there is a sophisticated one available for R. The package is called plotly. The last two lines of the code in mainBivariateSampling.R display an interactive 3D plot in the Viewer panel, which uses color to code the third dimension. It is interactive in the sense that by dragging the cursor over the plot one can visualize it from different viewpoints. Since the book will not appear in color one needs to run the code and get familiar with it. The reader should change the parameters (within reasonable limits) to better understand the example. The examples shown below are for the following choices:

.

| (A) | (B) |
| --- | --- |
| (C) | (D) |

Figure 21.1  (a): the default view;  (b): "birds-eye" view; (c): view along the y-axis at x = 0;  (d): view along the x-axis at y = 0. These plots were generated by the last two lines of the code in mainBivariateSampling.R.

Figure 21.1 (a) is the default view. Figure 21.1 (b) is the "birds eye" view, with the pdf-axis "literally coming at the reader" at right angles to the paper or screen. Figure 21.1 (c) is the view looking along the y-axis at x = 0. Figure 21.1 (d) is the view looking along the x-axis at y = 0. If the cursor is placed inside the plot area, plotly displays the three coordinates (x,y and the pdf) and slices along the two primary axes, as well as contours connecting all points with the same pdf value. These capabilities cannot be appreciated from the printed figure and the above description – one has to experience them.

# 21.5: Estimating bivariate binormal model parameters

In Chapter 06, a method for estimating the parameters of the univariate binormal model for a single-reader single-modality dataset was described. The method involved maximizing the likelihood function, i.e., the probability of the observed data as a function of the model parameters. Specifically, for a 5-rating study, there were four thresholds. The likelihood was a function of six variables: two basic parameters, namely the separation of the two normal distributions and the standard deviation  of the diseased distribution relative to the non-diseased distribution. In addition, there are the four thresholds. The threshold parameters are sometimes referred to, in the literature, as nuisance parameters, but nuisance or not, they have to be estimated in order to get to the basic parameters. To estimate the parameters the likelihood function was maximized with respect to these parameters. The values of the parameters yielding the maximum are the maximum-likelihood estimates (MLEs).

With a bivariate model, one is dealing with six basic parameters  and threshold parameters corresponding to each modality. The estimation procedure is not fundamentally different, but the presence of the second dimension does complicate the notation. Again, the starting point is the likelihood function, i.e., the probability of the observed data as a function of the parameter values. Since there is a possibility of confusion between the truth index t and the modality index i, each of which ranges from 1 to 2, the non-diseased counts in bin r of the 1st modality and bin s of the 2nd modality is denoted by  (i.e., the two modality indices occur before the truth index). The corresponding diseased counts are denoted . One needs two indices  because each case yields two integer ratings, r and s.  Assume R ratings in the first modality and S ratings in the second modality. To construct the matrix  one starts with a zero-initialized R x S matrix, increment the cell at row r and column s of this matrix by unity for each non-diseased case that received a r rating in the 1st modality and an s rating in the 2nd modality. When all non-diseased cases are exhausted, the count in row r and column s of this matrix is ; the procedure is repeated for the diseased cases yielding .

For non-diseased cases, the probability of a Z-sample in bin r of the 1st modality and bin s of the 2nd modality is determined by the “volume” under the bivariate distribution between modality-1 thresholds  and , and between modality-2 thresholds  and . Complicated as this may seem, and prior to Alan Genz’s work2 showing how to evaluate such multidimensional integrals, this was near impossible, but now it is quite easy, especially in R. The required probability is given by the pmvnorm() function – this is the generalization to the multivariate case of the pnorm() function, which gives the area under the normal distribution from negative infinity to a specified value. Specifically, pmvnorm()gives the probability under the multivariate distribution between specified thresholds, as illustrated next.

## 21.5.1: Examples of bivariate normal probability integrals

Needed is the bivariate extension of the univariate cumulative distribution function  defined in Eqn. (3.7). Specifically, one needs a way of calculating the integral:

.

Online Appendix 21.C.1 shows how to do this in R. The code file mainBivariateIntegrals.R implements the "code" equation:

.

Here pmvnorm() is the integral of the bivariate normal distribution with specified mean and covariance matrix sigma between limits  and . The limits are specified by two length-two arrays that are supplied as the first two arguments to this function. The first argument to the function represents the concatenated lower thresholds in the two modalities. The second argument represents the concatenated upper thresholds in the two modalities. The separation parameter vector and the covariance matrix follow these arguments. The function is not limited to two dimensional covariance matrices. With a p x p covariance matrix, the lower and upper limits would each be length p vectors. In lines 27-28, Online Appendix 21.C.1,  and . Sourcing the code yields:

### 21.5.1.1: Examples of bivariate normal probability integrals

> source(...)

Integrals of the bivariate normal ...

Over the entire space =  1

Over the full space in one dimension and the -ve half space in the other dimension =  0.5

Between specified ctff. values =  0.001136472

The facts that the integral over the entire space yields unity, i.e.,  and  and that over each half space yields 0.5, i.e.,  and , should make intuitive sense.

## 21.5.2: Likelihood function

The probability of observing and  non-diseased and diseased counts, respectively, in bin r in the first modality and bin s in the second modality is:

.

Including all bins and taking the logarithm, the logarithm of the likelihood function is given by (neglecting a combinatorial factor that does not depend on the parameters):

.

The maximum likelihood estimates of the parameters are obtained by maximizing the  function. This was solved 3,4 by Metz and colleagues more than three decades ago and implemented in software CORROC2. The Metz software is particularly useful as it measures correlations at the underlying Z-sample level, which property can be used in designing a data simulator that is matched to a clinical dataset, the subject of the next chapter. Much as the author has emphasized that ratings are not numerical quantities, and that the observed ratings are not "hard" numbers, the measured correlations according to this algorithm are valid, because CORROC2 models the ratings as continuous variables and estimates the correlation based on the bivariate model.

# 21.6: CORROC2 software

Around 1980 Prof. Charles E. Metz and Helen B. Kronman developed software implementing maximum likelihood estimation of the parameters of the bivariate normal model outlined above from data corresponding to two correlated ROC ratings per case. The software is called CORROC2, for “correlated ROC". Subsequent revisions to the program were made by Pu-Lan Wang and Jong-Her Shen and more recently by Ben Herman3-6. CORROC2 is used to analyze paired ratings that result when a common set of cases is interpreted under two conditions. The pairing can be simple, as when a single reader interprets a common set of cases in two modalities, or more general as when two different readers interpret a common set of cases in one modality, or when two readers interpret a common set of cases in two different modalities. The generality makes the method quite useful in a way that, in the judgment of the author, has not been fully exploited to date.

CORROC2 software allows selection of three methods of testing for differences between two conditions.

- Testing for simultaneous differences between the (a, b) parameters corresponding to two ROC curves.

- Testing the difference between the binormal-fitted AUCs under two correlated ROCs.

- Testing the difference between TPFs on two ROC curves at a specified FPF.

To understand the difference between (1) and (2) consider that is that it is possible to have two ROC curves with different shapes but the same fitted AUCs. If the difference in shapes is large enough, method (1) would declare the two conditions different, but method (2) would not. In the examples below, we use method (2).

## 21.6.1: Practical details of CORROC2 software

This section is one of the few in this book that uses Windows code. If using OS X, one needs to download "virtual machine" software as described in Online Appendix 21.D. If using a Windows machine, one does not need to do anything extra. Therefore, in following this chapter it is important that one has either a Windows computer or a Mac computer with a virtual machine installed.

The Windows executable code is named CORROC2.exe. The author downloaded the software, including the Fortran source code, many years ago, ca. 1997, from the University of Chicago website, but it is currently unavailable and unsupported on that site. The author is fortunate that he held onto the downloaded code. The reason is that the FORTRAN source code, with appropriate modifications, can be interfaced to the R code. The currently distributed equivalent code named ROC-kit cannot be run in command-line mode (i.e., without having to use the mouse to select the input file, specify the output file, etc.) The author has confirmed with Mr. Ben Herman, who is familiar with the code, that by design it cannot be used in command-line mode. The ability to run in command-line mode is needed if one is to call a program from R.

For simplicity, it is assumed that the number of allowed ROC ratings bins, R, is the same in each modality (the software can handle different numbers of ratings in the two modalities). For two modalities, the total number of parameters to be estimated is 6+2(R-1). The six comes from the two means and two variances of the diseased distributions, plus the two correlations (one per truth state) and 2(R-1) from the two sets of thresholds, each with R-1 values. In addition to providing estimates of all parameters of the model, the algorithm provides estimates of the covariance matrix of the parameters. This, not to be confused with , is needed for the same reason it was needed in the univariate case, see Chapter 06: i.e., to estimate variances of the 6-estimates and variances of functions of estimates, e.g., AUC or differences in AUCs.

## 21.6.2: CORROC2 data input

Here is a partial listing of how the paired ratings data is input to the program (the data file is DataFileInp.txt in the software distribution under software/corrocii/Debug).

TYPICAL EXAMPLE OF 5-CATEGORY DATA --X

5

5

TYPICAL EXAMPLE OF 5-CATEGORY DATA --Y

5

5

4 4

3 5

5 3

1 1

1 1

1 3

...

The first line, whose content is immaterial to the analysis, is a description of the data in the 1st modality. The second line (5), is the total number of ratings bins in the 1st modality. The third line (5) is the rating with the highest evidence of disease in the 1st modality (5 in the current example). The fourth line is a description of data in the 2nd modality, again its content is immaterial. The next two fives are the number of bins in the second modality, and the bin number denoting highest evidence of disease in the second modality.

The actual ratings data starts on line 7, which states that the first non-diseased case was rated ROC: 4 in the first modality and ROC: 4 in the second modality. The two ratings for each case, separated by spaces, are input on the same line. Line eight states that the second non-diseased case was rated ROC: 3 in the first modality and ROC: 5 in the second modality, and so on. An asterisk after the last non-diseased case signals the end of the non-diseased cases.  The next line has the two ratings corresponding to the first diseased case. Here is a partial example:

4 4

*

4 5

5 5

This says that the first diseased case was rated ROC: 4 in the 1st modality and ROC: 5 in the 2nd modality, the 2nd diseased case was rated ROC: 5 in both modalities, and so on. The last diseased case is followed by another asterisk, and the last line is the word “area”:

5 5

2 5

*

area

Other choices are available for the last line, but the author will stick to area, which means one wishes to use the area under the binormal-fitted ROC curve  as the figure of merit.

## 21.6.3: CORROC2 output

The CORROC2 output associated with this software for the above dataset, 347 lines in all, is in the file DataFileOut.txt, Fig. 21.2 (use RStudio to go down the directory structure and click on the appropriate file; it is displayed, see Figure 21.2 below, with line numbers):

Fig. 21.2: Lines of CORROC2 output in DataFileOut.txt that are associated with the dataset in DataFileInp.txt.

Condition X and Condition Y denote modalities 1 and 2, respectively. Line 10 of the output summarizes the statistical test to be employed “AREA (A SUB Z) TEST”. This is because we ended the reader data file DataFileInp.txt with area. Line 13-22 summarizes the dataset, how many ratings, which rating represents greatest evidence of disease, etc. Lines 26 – 37 summarize the observed ratings matrix for non-diseased cases (labeled ACTUALLY NEGATIVE), i.e., the matrix . For example, there were three cases that were rated s = 3 in CONDITION X and r = 5 in CONDITION Y. The matrix is labeled by its row index, followed by its column index and last with the truth index. The output lists the 5-rating bin as the first row, the 4-rating bin as the second row, etc. Likewise, lines 40-51 summarize the ratings matrix  for diseased cases. Lines 53 – 59 list the observed operating points in the two modalities, followed by the initial and final estimates of the parameters. The program lists the parameters using the (a,b) notation, not the  notation, but one knows how to transform between them: Chapter 06, Eqn. (6.16), and see exercise on conversion below. Here is the output showing the final parameter values (14 values in all starting at line 74):

FINAL ESTIMATES OF THE PARAMETERS:

AX= 1.3006     BX= 0.4919     AY= 1.6049     BY= 0.8704

R(NEGATIVE CASES)= 0.2599     R(POSITIVE CASES)= 0.4544

T(I)-0.281  0.221  0.687  1.941

U(J)-0.791 -0.417  0.175  0.990

Here X refers to condition X and Y to condition Y. AX is the  parameter in condition X (what the author calls ). BX is the b parameter in condition X (our ). AY is the  parameter in condition Y (i.e., ). BY is the b parameter in condition Y (i.e., ). R(NEGATIVE CASES) is the correlation coefficient for the non-diseased cases (i.e., ). R(POSITIVE CASES) is the correlation coefficient for the diseased cases (i.e., ). Finally, T(I) and U(J) are the thresholds in the two condition, corresponding to  and  in our notation. Since there are five ratings bins, there are four thresholds in each modality.

### 21.6.3.1: Exercise: convert the above parameter values to  notation

Using the transformations Eqn. (6.16), = 1.3006/0.4919 = 2.644, = 1.844, = 1/.4919 = 2.033, = 1.149, = 0.2599 and = 0.4544 (the correlations are unaffected by linear transformations). Regarding the thresholds, for modality 1, = -0.281/.4919 = -0.571, = 0.221/.4919 = 0.449, etc. Similarly, the thresholds for modality 2 are = -0.791/.8704 = -0.909, = -0.417/.8704 = -0.479, etc. Question for the reader: why were the thresholds divided by the corresponding b-values?

## 21.6.4: Covariance matrix

For convenience of the yet-to-be-described R-program that does all this automatically, the FORTRAN source code was modified to print the elements of the covariance matrix sequentially, i.e., one-per-line. The output corresponding to lines 83 - 280 in file DataFileOut.txt is (shown is a partial listing):

VARIANCE-COVARIANCE MATRIX

14

0.621003E-01

0.205564E-01

0.171881E-01

0.294564E-02

0.104652E-03

-0.684784E-02

0.144067E-01

0.135140E-01

0.127711E-01

0.393710E-02

0.285687E-02

0.285592E-02

0.285248E-02

0.284967E-02

0.205564E-01

The "14" is the dimension of each "edge" of the 2D covariance matrix, i.e., there are 142 (196) elements in all. The "14" comes from the six basic parameters plus the four thresholds in the two conditions. The elements are listed in row-major order, i.e., all the elements of the 1st row, followed by all elements of the 2nd row, etc. One can anticipate some difficulty displaying it, so only the upper-left 6 x 6 corner of the full matrix is shown below, Table 21.1, i.e. any covariance involving a threshold is not shown. For convenience, the numbers have been rounded to fit and since the matrix is symmetric, there is no need to show numbers below the diagonal. The diagonal elements, which represent variances, are all positive.

Table 21.1: This table displays the covariance matrix of the parameters for the dataset in datafileinp.txt. Since the matrix is symmetric, values below the diagonal are not shown. Also not shown are covariance-elements associated with threshold parameters.

|  | .062 | .021 | .017 | .003 | .000 | -.007 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | .022 | .002 | .002 | .001 | -.004 |
|  |  |  | .089 | .045 | .000 | -.010 |
|  |  |  |  | .078 | .003 | -.008 |
|  |  |  |  |  | .026 | .000 |
|  |  |  |  |  |  | .035 |

## 21.6.5: Rest of CORROC2 output

Line 286-320 allows one to plot the ROC curves for the two modalities, but one does not need them, as one knows by now how to generate binormal model predicted curves. Lines 325-327 lists the areas under the respective fitted ROC curves, and their correlation (the is the AUC level correlation, which could be estimated by the methods described in Chapter 10):

AREA(X)=  0.8784     AREA(Y)=  0.8870

STD DEV OF AREA(X)=  0.0392     STD DEV OF AREA(Y)=  0.0376

CORRELATION OF AREA(X) AND AREA(Y) = 0.2569

The remaining lines, 333-341 lists the p-value.

STATISTICAL SIGNIFICANCE OF THE DIFFERENCE

BETWEEN THE TWO CORRELATED ROC CURVES ACCORDING TO THE SELECTED TEST:

*****************************************************************************

THE COMPUTED CORRELATED 'AREA TEST' STATISTIC VALUE IS   -0.1829

WITH A CORRESPONDING TWO-TAILED P-LEVEL OF 0.8549

AND ONE-TAILED P-LEVEL OF 0.4274.

The famed p-value is 0.8549 (it is customary to quote the two-tailed value, see Chapter 08 for details on this). This means that the difference in areas (0.8784 – 0.8870 = -0.0086) is not statistically significant at Type-I error probability  = 0.05. The calculation of the test statistic is described in code mainDiffAzTestStatistic.R. It uses methods described in Chapter 06 to calculate the standard deviation of the difference of two AUCs:

.

The test – statistic for determining the significance of the AUC difference is:

.

# 21.7: Application to a real dataset

Assuming one has set up the connection between the Mac side and the Windows side properly, as described in Online Appendix 21.D, one can apply CORROC2 to real ROC datasets. Open the R file “mainCorroc2.R” in the software folder corresponding to this chapter, described in detail in Online Appendix 21.E. Source the file, yielding:

### 21.7.1: Code Output

> source(...)

...

The 6 parameters are  1.72 0.55 1.87 0.527 0.332 0.798

The 2 sided pValue is  0.482

The covariance matrix is:

[,1]     [,2]      [,3]     [,4]     [,5]      [,6]

[1,]  0.120075  0.05096  0.050939  0.01054 6.58e-04 -1.05e-02

[2,]  0.050956  0.03667  0.013392  0.00550 2.01e-03 -4.87e-03

[3,]  0.050939  0.01339  0.111921  0.04078 2.28e-04 -1.16e-02

[4,]  0.010545  0.00550  0.040777  0.02908 1.51e-03 -4.56e-03

[5,]  0.000658  0.00201  0.000228  0.00151 2.09e-02  3.11e-06

[6,] -0.010453 -0.00487 -0.011566 -0.00456 3.11e-06  8.67e-03

What the R code does is detailed in Online Appendix 21.D. It reads the file containing the data, the data in the above example, extract the ratings for reader 1 in both modalities, writes the data in the appropriate format for CORROC2, runs the latter program in a Windows environment, reads the resulting output and prints out model parameters and the covariance matrix.

Like most ROC software, the output uses the (a,b) convention, Chapter 06. Recall that in this convention the diseased distribution has unit standard deviation, the non-diseased distribution has standard deviation b, and the separation of the two means is a. Since there are two modalities, there are two a parameters and two b parameters. In the CORROC2 output, the two modalities are referred to as X (1 in our notation) and Y (2 in our notation), respectively, and the ordering of the six parameters is . The ordering also determines the ordering of the covariance matrix. We start with the meanings of the header row and first column of the matrix shown in §21.7.1. The notation [,1] means the first column of the matrix, likewise [,2] is the second column and so on. Likewise, [1,] means the first row of the matrix, [2,] means the second row of the matrix, and so on. For example, the above table tells us that the second row and third column of the covariance matrix is 0.013392. This is the covariance between  and i.e., . The diagonal elements of the matrix are the variances. For example, the variance of  is 2.09e-02, whose square root is the standard deviation, equal to 0.145.

In the Files panel find the folder named corrocii and open it (the reader needs to click on the name, not the picture of the folder; watch how the folder name is underlined when the cursor is above it, which is RStudio’s way of telling the reader “yes, it is OK to click here, something will happen”). One should see a number of files with the .FOR extension (these are FORTRAN source code files) and a folder named Debug; click on Debug. Fig. 21.3 is a screen-shot showing the contents of the Debug window. Table 21.2 summarizes the meanings of the files listed in this figure.

Fig. 21.3: Contents of the Debug window, as viewed from the Mac side.

Table 21.2: This table summarizes the meanings of the files listed in Fig. 21.3.

| File name | Meaning | Comment |
| --- | --- | --- |
| DataFileInp.txt | Input Ratings data in CORROC2 format | Read-only file used to illustrate data input format |
| DataFileOut.txt | Output file corresponding to DataFileInp.txt | Read-only file used to illustrate the output |
| 1R2MData.txt | Ratings data in CORROC2 format | Created by RunCorrocOnPairedData.R, line 25 |
| CORROC2.bat | Batch file that runs CORROC2.exe | See below |
| CORROC2.exe | The actual CORROC2 program |  |
| CorrocIIinput.txt | Input file to the batch file |  |
| CorrocIIoutput.txt | Output file created by batch file |  |

The Windows software is located in software/CORROC2/Debug, Fig. 21.4. CORROC2.bat is a batch file, whose listing follows:

### 21.7.2: Contents of CORROC2.bat

cd CORROCII

cd Debug

del CorrocIIoutput.txt

CORROC2.exe < CorrocIIinput.txt > CorrocIIoutput.txt

cd ..

cd ..

The batch file runs on the Windows side. The first two lines change the current directory to corrocii/Debug. The next line deletes the output file, CorrocIIoutput.txt, if it exists. The next line "pipes" the contents of CorrocIIinput.txt to the executable file CORROC2.exe and "pipes" the output to CorrocIIoutput.txt. The last two lines restore the starting directory. The contents of CorrocIIinput.txt are:

### 21.7.3: Contents of CorrocIIinput.txt

y

1R2MData.txt

N

Q

These answer the four questions asked by CORROC2. Try running CORROC2 from the Windows side, Fig. 21.4.

Fig. 21.4: Result of running CORROC2 at the Windows 8 command prompt.

The first line of this file is the answer (yes) to the first question asked by CORROC2.exe (the reader can check these out by clicking on CORROC2.exe in the reader's physical Windows machine or virtual Windows machine): “Do you want to use data from a previously created input file for the next run?” The second line is the name of the input file, the third line is the answer (no) to the next question: “Do you want to save an output file for plotting the ROC curve?” The reason for the "no" is that one is, for the present, more interested in the screen output, which contains all the estimates, including the covariance matrix, rather than plotting ROC curves. The last line is the answer ("Q") to the next question asked by CORROC2.exe: “Do you want to use data from a previously created input file for the next run? (Y/N, or Q for quit)”. Now, in a Windows environment, click on the batch file CORROC2.bat. The latest version of the file CorrocIIoutput.txt should appear in the readers Explorer window, with the correct date and time stamps. Open this file. The data file is named 1R2MData.txt and if you open it, you will recognize that it is similar to the file used in the section illustrating the data input format for CORROC2. It was created by the R-code.

# 21.8: Discussion / Summary

CORROC2 is one of the relatively under-utilized tools developed by the late Prof. Charles E. Metz. There are only four publications describing it3-5,7 and two of them are difficult to find4,7, nor has the software been maintained on a level comparable to ROCFIT. For example, a method for assessing the goodness of the fit is currently not implemented. With RxS cells, it is almost impossible to maintain greater than five counts per cell, so an alternate method of combining bins needs to be implemented.

One reason for the relative underutilization of CORROC2 could be that it is not designed to analyze multiple readers interpreting a common set of cases in two or more modalities i.e., MRMC datasets. For example, in a study in which CORROC2 was used8, four readers interpreted cases in four modalities. With four modalities, there are six possible combinations (4x3/2), so a conservative Bonferroni type correction for the p-value would be to divide 0.05 by 6. For each reader CORROC2 yields a p-value for the difference between the chosen pairs of modalities. If all readers agree, then there is an unambiguous answer, but the method does not allow for correlations between different readers, which undoubtedly leads to loss of power. More fundamentally, it does not address the question of interest: when averaged over all four readers, are the figures of merit in the four modalities different.

It is possible to analyze multiple reader multiple modality MRMC-ROC datasets without actually using CORROC2, as was done in Chapter 09 and Chapter 10. The correlations in the resulting AUC values – i.e., FOM- correlations as distinct from ratings-correlations - are explicitly accounted for in the DBMH / ORH analyses, i.e., the described methods are not concerned with rating correlations. One possible use of CORROC2 would be if one had to compare two CAD algorithms applied to a common dataset, but even here, it is of limited utility since, as a ROC study, location is ignored.

CORROC2 was developed ca. 1980 by Metz, Wang and Kronman 4,6. Subsequent revisions to the program were made by Jong-Her Shen and more recently by Benjamin Herman5,6. There are in excess of 116 citations to this software. One reason is that, at that time, it was the only software allowing analysis of paired datasets. However, no advances have been made in the intervening 3 decades, which would allow fitting, for example, proper ROC curves to paired - and possibly degenerate - datasets. Part of the reason for this neglect is the shift in emphasis to empirical AUC based analysis, which does not require parametric modeling or curve fitting.

The real use of CORROC2 is if one is interested in ratings correlations. In fact, until recently, it was the only way to estimate the ratings correlations. Interest in the correlations of ratings arises if one wishes to design a data simulator that is calibrated to a given dataset. Roe and Metz performed some work along this line9 when a set of ratings simulators were suggested as representative of real ROC datasets. These simulators have been widely used in the methodology literature to validate analysis methods. While simulators are of primary use to methodology developers, their impact is much greater than their seemingly limited applicability. Part of the unfinished business of this book is to develop a calibrated simulator. Initial progress in this direction was stymied by the fact that binormal model based CORROC2 cannot handle degenerate datasets. Progress in using a correlated version of CBM is currently underway. It should be robust to degenerate datasets and yield proper ROC fits in each modality.

At the time of writing, 4/30/17, an extension of CORROC2 has been published10. It is called CORCBM, for correlated CBM. Details are in a document CORCBM.pdf in the online supplementary material directory corresponding to this chapter. The title of the publication is: "A bivariate contaminated binormal model for robust fitting of proper ROC curves to a pair of correlated, possibly degenerate, ROC datasets". It replaces the bivariate binormal model with the bivariate contaminated binormal model; hence its name CORCBM, for correlated CBM. Since CBM was designed to fit practically any single dataset, including degenerate ones, CORCBM is likewise able to fit practically any paired dataset. An application of CORCBM to calibrating a simulator to a data set containing single-modality multiple-reader ratings is described in Chapter 23.

# 21.9: References

1.	Tong YL. The Multivariate Normal Distribution. first ed. New York: Springer- Verlag; 1990.

2.	Genz A. Numerical computation of multivariate normal probabilities. Journal of computational and graphical statistics. 1992;1(2):141-149.

3.	Metz CE, Kronman H. Statistical Significance Tests for Binormal ROC Curves. J Math Psychol. 1980;22(3):218-242.

4.	Metz CE, Wang P-L, Kronman HB, eds. A New Approach For Testing the Significance of Differences Between ROC Curves Measured from Correlated Data. The Hague: Nijhoff; 1984. Deconinck F, ed. Information Processing in Medical Imaging.

5.	Metz CE. Some Practical Issues of Experimental Design and Data Analysis in Radiological ROC studies. Investigative Radiology. 1989;24:234-245.

6.	Metz CE. ROCKIT Beta version, available from http://www-radiology.uchicago.edu/krl/ University of Chicago; [updated 2007 Feb 27; cited 2008 Oct]. . 1998.

7.	Metz CE, Kronman H. A Test for the Statistical Significance of Differences Between ROC Curves. INSERM. 1979;88:647-660.

8.	Farquhar TH, Llacer J, Hoh CK, et al. ROC and Localization ROC Analyses of Lesion Detection in Whole-Body FDG PET: Effects of Acquisition Mode, Attenuation Correction and Reconstruction Algorithm. J NucI Med. 1999;40:2043 - 2052.

9.	Roe CA, Metz CE. Dorfman-Berbaum-Metz Method for Statistical Analysis of Multireader, Multimodality Receiver Operating Characteristic Data: Validation with Computer Simulation. Acad Radiol. 1997;4:298-303.

10.	Zhai X, Chakraborty DP. A bivariate contaminated binormal model for robust fitting of proper ROC curves to a pair of correlated, possibly degenerate, ROC datasets. Medical Physics. 2017:2207–2222.

The author recalls a conversation with the late Prof. Richard (Dick) Swensson at an SPIE, where Dick enthusiastically explained a project he was working on that utilized CORROC2 repeatedly.

"Condition" is a more general term than modalities; as already noted the pairing could be reader 1 vs. reader 2 in modality 1, or even reader 1 in modality 1 vs. reader 2 in modality 2; all that is required for a valid pairing is interpretations of a common set of cases.

All this will not make sense unless one views the output; obviously the full output cannot be shown in the book, but one can view it using the downloaded software.